Batteries

- Pallet 1 (46x45x54", 527 LBs)

  - Zarges Box 3

    - Radon Instrument: Nonspillable battery installed in equipment. “NONSPILLABLE” label required. Must list on AWB:

      - “Not Restricted per Special Provision A67” on the airwaybill for the battery in here.

  - Zarges Box 5

    - Rechargeable AA Batteries: Not restricted. SDS received. No label needed. Must have on the airwaybill:

      - “Not Restricted per Special Provision A199”

- Pallet 2 (52x48x52", 824 LBs)

  - Zarges Box 4

    - Zonge Battery: Sealed nonspillable battery. SDS provided. Label Box 4 with a “NONSPILLABLE” label. Need to list on the AWB:

      - Not Restricted per Special Provision A67”

    - UPS Battery: Sealed nonspillable battery. SDS provided. Additional reminder to label this Box 4 with a “NONSPILLABLE” label. Need to list on the AWB:

      - “Not Restricted per Special Provision A67”

    - Alkaline 62SP Battery Pack, D cell Alkaline Batteries, & C cell Alkaline batteries: No label required. Must list on the airwaybill:

      - “Not restricted per special provision A123”

    - Camera Gimbal Lithium Ion Battery: UN3481 Lithium Ion Battery Contained in Equipment. Only 21.6 Watt-Hours and 0.1 KGs. PI 967 Section II. Do NOT need to list on the dangerous goods declaration. Lithium battery mark required. Must note on the AWB:

      - “Lithium Ion Batteries in Compliance with Section II of PI 967”

  - Large CSEM Milwaukee Toolbox 3:

    - Three Milwaukee Tools with Lithium Ion Batteries Packed with Equipment: UN3481 Lithium Ion Batteries Packed with Equipment. Received SDSs. One impact drill, one drill and one flashlight. There’s a 2aH (M12 B2) battery for the impact drill, 3aH (M12 B3) battery for the regular drill, and a 4aH (M12 B4) for the flashlight. These are all UN3481 Lithium Ion Batteries Packed WITH Equipment. PI 966, Section II. All three batteries have a watt hour rating of less than 100. The B2 battery weighs 0.2 KGs and a Wh rating of 24, the B3 battery also weighs 0.2 KGs and has a Wh rating of 36, and the B4 battery weighs 0.4 KGs and has a Wh rating of 48.

      - “Lithium Ion Batteries in Compliance with Section II of PI 966”

- Pallet 3 (46x46x36", 300 LBs, Overpack Used)

  - Black Husky Box:

    - BlueROV with UN3481 Section I: UN3481 Lithium Ion Batteries Contained in Equipment. PI 967 Section I.

  - Note: The other box on this pallet contains no dangerous goods.