5th December 2024

Team: Johann Voitglander, Roman Marin, Eric Fitzgerald, Ben Jansen, Charlie Paull, Roberto Gwiazda, Aaron Micallef

Data loggers:

10 kHz

TX1: 159 (new battery)

TX2: 160 (new battery)

Boat: 162

| Time (local) | Activity |
| --- | --- |
| 9.12 a.m. | Start deployment, 1.5 kn |
| 9.14 | Tail buoy in water |
| 9.16 | Stopped deployment to attach first data logger |
| 9.38 | Second buoy in water |
| 9.39 | First data logger in water |
| 9.47 | Data logger back on board due to issues with lines |
| 9.49 | Data logger back in water |
| 9.55 | Data logger recovered again |
| 10.01 | Data logger in water |
| 10.26 | Second data logger in water, buoy in water |
| 10.40 | Captain following 15 m isobath |
| 10.42 | Switch from blue to black winch |
| 10.45 | First metal sphere attached |
| 10.50 | First metal sphere in water, buoy in water |
|  | TSUNAMI WARNING |
| 11.04 | Second metal sphere attached |
| 11.05 | Second metal sphere in water |
| 11.14 | Payout completed (0.5 nm from starting point) |
| 11.16 | Start survey at 36°53.428 N, 121°50.785 W, 1.7 kn |
| 3.15 p.m. | Started slight turn to avoid a buoy. Blue cube at 40°C |
| 3.45 | End survey at 36°57.306 N, 121°56.868 W |
| 3.54 | Black cable (connected to transmitter) wrapped around black winch |
| 3.57 | Started recovery of thick black cable |
| 4.01 | First sphere draining |
| 4.03 | First sphere on deck |
| 4.10 | Recovery at 0.5-0.8 kn |
| 4.16 | Second sphere draining |
| 4.19 | Second sphere on deck |
| 4.24 | Switch from black to blue winch |
| 4.40 | First data logger on deck |
| 4.47 | Second buoy of first data logger on deck |
| 5.04 | Second data logger on deck |
| 5.09 | Second buoy of second data logger on deck |
| 5.14 | Tail buoy on deck, recovery complete |

Suggestions: add radar reflectors, increase depth of data loggers