Setup

- Spectra lengths spooled onto blue winch with tie-back line through final shackle and secured to drum. (See “BlueWinch_V1.pdf”)

- Two receiver arrays staged with weight-end available first, and logger end available last.

- Spiral-wrap section spooled onto black winch, with tieback line secured to drum.  (See “BlackWinch_V1.pdf”)

- Transmitter clamps secured to rope on drum (alternatively these can be attached during deployment.

- Two transmitter sphere's available and prepped.

- Winches installed on boat and two 8m slip lines available with cleats, one on each side.

Deployment

- Set up 3 miles from the start of the survey line, it should take an hour and a half to deploy the string. Approach the start point at about 1-2 knots while paying out the instrument string

- Starting with the blue winch, deploy the terminal float and pay out the first 35 meters until the first friction rings and lanyards are off the winch.

- Attach the first receiver array to the friction rings and lanyards while paying out.  It shouldn’t be necessary to stop of the string while doing this.

- Pay out the next 330m of 1/8” spectra and then attach the second receiver array in the same manner as the first

- Pay out the remaining 250m of ¼” Spectra until the final stainless thimble and soft-shackle are free of the blue winch.  Stop this off with a small diameter slip line that is at least 8m long.

- Connect the free end of the spiral section to the soft-shackle and disconnect the blue-winch tie back rope.

- Slip out string until the first transmitter clamps are free of the black winch. Install transmitter and connect cabling.

- Slip string further out until load is taken by the black winch and remove slip line.

- Deploy transmitter overboard and pay out 120m of spiral-wrapped line.

- Stop off at thimbles with slip line and install second transmitter.

- Slip out line and deploy second transmitter

- Pay out 50 m of spiral-wrapped line and until final stainless thimble is free of the black winch.

- Install two securement lines to stainless thimble and pay out black winch until the stainless thimble is just aft of the boat.

- Connect the electrical conductors to instrumentation and proceed with the measurement

Recovery

- Disconnect electrical connectors and wind these back on black winch drum.

- Reel in spiral wrap onto the black winch and remove stop-off lines.

- Recover first transmitter onto the deck and stop off on thimbles.  Disconnect transmitter

- Reel in until the second transmitter is on the deck. Stop off on yale-grip and disconnect transmitter

- Reel in on black winch until transition to spectra line is on board.  Stop off as needed and re-attach blue-winch tie-back line to spectra.

- Reel in 250m of ¼” spectra until first receiver array comes aboard.  Disconnect lanyards one by one as string comes aboard.

- Reel in 330m of 1/8” spectra until second receiver array comes aboard.  Disconnect lanyards one by one as string comes aboard

- Recover remaining line and distal float.