Energy Levels of SWAN system

1. Logic Power:

- Logic supply: 9 to 140V

- Peak current draw: 1.6A

- Average current @ 12V: <1 ampere

2. Input Power:

- Touch Proof SurLok PlusTM Connectors

- 11 to 600 volts

3. Transmitter Control:

- 3 - 14mA current drive (15mA absolute maximum)

- Optical isolation

4. Output:

- Peak current: 30 A unregulated

- 11 to ~600 V

- Over-current shutdown (set for ~33 A)

- IGBT current switch

- Power removed from transmitter output during fault conditions

5. Transmitter Frequency:

- DC to 128 Hz time domain

- DC to 8196 Hz frequency domain

6. Current Monitor:

- 4mm Sheathed Banana Jack

- Isolated output

- 10mV/A

Case 1: frequency domain acquisition

To calculate the energy consumed by the system considering the following operating setting:

- Voltage ( V ) = 30 V

- Current ( I )= 25 A

- Operative time per cycle  = 180 s

- Stop time per cycle = 60 s

- Duty cycle = 100%

- Total time = 6 hours

The energy levels of the system would be 3375 watt-hours.

Case 2: time domain acquisition

To calculate the energy consumed by the system with a 50% duty cycle and a source frequency of 4 s, considering a voltage of 30 V and a current of 25 A, the energy level of the transmitting system would be 93.75 watt-hours.