Assembly Notes for Upwelling Wago Enclosure

Wiring Colors

24Vdc: Violet

5Vdc: Red

GND: Black

Chassis (Shield): Green

110VAC_LINE: Black

110VAC_NEUTRAL: White

110VAC_CHASSIS: Green

Wire Gauges

Unless noted otherwise, all signals are 22AWG wire.

Known exceptions are:

The 110VAC signals: 18AWG

Wire Bundles

A

Type: RTD (Temperature) Signals

Cable: Alpha Wire PN# 6010 SL005  [TP (x3) 22AWG]

Number: 16

From: Junction Box Connectors (J1-J16)

To: Wago Stack (750-460s)

B

Type: Heater Control Signals

Cable: Alpha Wire PN# 5481C [TP (x1) 22AWG]

Number: 32

From: Wago Stack (750-1504s) and 24Vdc RETURN TBs (280-651s)

To: 220VAC Box Connectors (J101, J102, and J103)

C

Type: pH Input Signals

Cable: Alpha Wire PN# 5481C [TP (x1) 22AWG]

Number: 32

From: pH Isolation Boards

To: Wago Stack (750-468s)

D

Type: pH ISO Signals

Cable: Alpha Wire PN# 5640B2201 [TT (x1) 22AWG]

Number: 32

From: pH Isolation Boards

To: Junction Box Connectors (J1-J16)

E

Type: O2 Signals

Cable: Alpha Wire PN# 5640B2201 [TT (x1) 22AWG]

Number: 32

From: Junction Box Connectors (J1-J16)

To: Wago Stack (750-468s)

F1

Type: Valve Power

Cable: Homemade twisted pairs (non-shielded, 18AWG)

Number: 2

From: 24Vdc and GND Terminal Blocks (280-651s)

To: Valve Controllers (Inner Door)

F2

Type: Valve Drive Signal

Cable: Alpha Wire PN# 6010 SL005  [TP (x3) 22AWG]

Number: 16

From: Wago Stack (750-559s)

To: Valve Controllers (Inner Door)

F3

Type: Valve Output Signals

Cable: Alpha Wire PN# 6010 SL005  [TP (x3) 22AWG]

Number: 16

From: Valve Controllers (Inner Door)

To: Junction Box Connectors (J1-J16)

Junction Box Connectors

| Pin | Signal | Cable Type | Bundle |
| --- | --- | --- | --- |
| A | Cx_PH_ISO_RET | 5640B - Twisted Triad | D |
| B | Cx_PH_ISO_SIG |  |  |
| C | Cx_PH_ISO_5V |  |  |
| D | SHIELD_C_PH |  |  |
| E | Cx_O2_RET | 5640B - Twisted Triad | E |
| F | Cx_O2_SIG |  |  |
| G | Cx_O2_5V |  |  |
| H | SHIELD_C_O2 |  |  |
| J | Ex_PH_ISO_RET | 5640B - Twisted Triad | D |
| K | Ex_PH_ISO_SIG |  |  |
| L | Ex_PH_ISO_5V |  |  |
| M | SHIELD_E_PH |  |  |
| N | Ex_O2_RET | 5640B - Twisted Triad | E |
| P | Ex_O2_SIG |  |  |
| R | Ex_O2_5V |  |  |
| S | SHIELD_E_O2 |  |  |
| T | Bx_T_SIG | 6010 SL005 - Twisted Pair (x3) | A |
| U | Bx_T_RET |  |  |
| V | Ex_T_SIG |  |  |
| W | Ex_T_RET |  |  |
| X | Cx_T_SIG |  |  |
| Y | Cx_T_RET |  |  |
| Z | SHIELD_TEMP |  |  |
| a | Vx_N2_POS | 6010 SL005 - Twisted Pair (x3) | F3 |
| b | Vx_N2_NEG |  |  |
| c | Vx_CO2_POS |  |  |
| d | Vx_CO2_NEG |  |  |
| e | Vx_O2_POS |  |  |
| f | Vx_O2_NEG |  |  |
| g | SHIELD_VALVE |  |  |

220VAC Box Connectors

| Pin | Signal | Cable Type | Bundle |
| --- | --- | --- | --- |
| A | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| B | Ex_H_RET |  |  |
| C | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| D | Cx_H_RET |  |  |
| E | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| F | Ex_H_RET |  |  |
| G | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| H | Cx_H_RET |  |  |
| J | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| K | Ex_H_RET |  |  |
| L | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| M | Cx_H_RET |  |  |
| N | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| P | Ex_H_RET |  |  |
| R | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| S | Cx_H_RET |  |  |
| T | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| U | Ex_H_RET |  |  |
| V | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| W | Cx_H_RET |  |  |
| X | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| Y | Ex_H_RET |  |  |
| Z | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| a | Cx_H_RET |  |  |
| b | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| c | Ex_H_RET |  |  |
| d | Cx_H_SIG | 5481C - Twisted Pair (x1) | B |
| e | Cx_H_RET |  |  |
| f | Ex_H_SIG | 5481C - Twisted Pair (x1) | B |
| g | Ex_H_RET |  |  |