CANON SIPPER SAMPLER API

Program Number: 901009

Prepared By: BDJ

Revision: 0.1

Date: JUL15

Contents

1.	Scope	3

1.1 Identification	3

1.2 Purpose	3

1.3 Acronyms and Abbreviations	3

2.	Applicable Documents	3

3.	Implementation	3

3.1 Communications setting	3

3.2 API	3

UI[1]=0<cr><lf>	3

XQ##Next()<cr><lf>	3

XQ##HOME()<cr><lf>	3

SV<cr><lf>	3

XQ##RunPump(int  x1)<cr><lf>	3

3.3 Normal Usage	3

Pre deployment	3

During deployment	4

# Scope

## 1.1 Identification

This document defines the application programing interface to the CANON SIPPER sampler

## 1.2 Purpose

## 1.3 Acronyms and Abbreviations

API – Application Programming Interface

# Applicable Documents

MAN-COMPUM.pdf – Elmo MC Inc. March 2007 Ver.1.9

MAN-DCWHIIG.pdf – Elmo MC Inc. July 2014 Ver.1.101

MAN-SIMCR.pdf – Elmo MC Inc. Feb 2010 Ver.4.5

Sipper Chassis Wiring – MBARI v.1.1.0

Sipper Whistle Adapter – MBARI v.1.1.0

# Implementation

## 3.1 Communications setting

RS-232 serial interface at 19200 baud, N81, no hardware/software flow control

## 3.2 API

### UI[1]=0<cr><lf>

Sets position to 0 (corresponds with sample chamber number) in flash.  Issue this BEFORE deployment.  Must also issue ‘SV’ command below.

### XQ##Next()<cr><lf>

Advance to next position (1-16).  Reads the current UI[1] value and advances to the next position.  Increments UI[1].  Issue a SV command if you want UI[1] to persist through power cycles.

### XQ##HOME()<cr><lf>

Rotate sampler to HOME position.

### SV<cr><lf>

Save new position number (UI[1]) in flash so it persists through power cycles

### XQ##RunPump(int  x1)<cr><lf>

Run hydraulic pump for x1 seconds.

## 3.3 Normal Usage

### Pre deployment

Issue the following commands before deploying

UI[1]=0

SV

### During deployment

Issue the following commands for each sample

XQ##Next()

SV

XQ##RunPump(seconds)