26SEP16 0500 local.

- I reflashed new program in CPF without opening pcase.

  - Only changed sed trap loiter timespan from 240 min to 215 min

- Power cycled CPF and visually checked for proper init.

- Measured Batt, VAC, RH, and BP.  All looked normal (didn’t record this anywhere)

- Rotated Sed trap and installed/filled bottles

- Power down.

26SEP16 0700 local

- Swiped magnet

- Watched CPF initialize properly (ran hyd motor and checked all sensors).  Unfortunately didn’t keep serial log file.

- Received SBD at 0702 from initial float try.  Reported Batt=32.72V, Vac=836.1, and BP=54.52.  Try to find RH

- Looked one last time at serial BT feed and it was in ABSETFAST state, which is normal

- Received SBD from iBCN at 0712.

- ~0715 deployed from WF via aft port crane.

  - Quick release system used

  - Did not appear to hit ship during transit down to water line

  - Release deployed without issue

  - Float immediately sank at what appeared to be a slow, normal rate

  - 36 53.99N 122 20.476W

26SEP16 0800 local

- Had not heard from float so began using dunker to range on the underwater acoustic modem.

- Successfully heard ranges from 670-770 meters

- Called in wave glider to locate modem

26SEP16 afternoon

- Ran Radius 400m results:

-Target position is lat:36.9005360636 lon:-122.342168294 depth:642.131444526

27SEP16 0920

1. Ran Radius 800m result:

  - Target position is lat:36.9005561947 lon:-122.34214752 depth:648.208724727

11OCT16 recovered with Ventana on dive 3970.

- Dropweight

  - Lead weight missing at the scene

  - Guiotinne fired.

  - Disk was .0165” thickness (pink)

  - Obviously fired at some point

- Implosion

- SD card recovered in mangled external case

  - McGill attempted to mount in new case and read.  No joy

  - Sent to Secure Data Recovery Services in Campbell, CA.

    - Case 101679

    - Could not recover any data because “monolythic chip was cracked in half”

- Main 10 amp fuse looks intact, like it didn’t burn

- Tore apart lower end and noticed high pressure hydraulics seemed sealed

  - Hydraulic pump showed no signs of water intrusion, nor corrosion on the inside

  - Hydraulic manifold showed no signs of water intrusion nor corrosion on the inside

  - External reservoir had dirt and water with ??? cc’s of oil.  Dirt and water ingress most likely during and after implosion.

- Top cap has damage on the side

  - Significant blow (see pic

- Pressure housing has damage on side opposite main implosion.  Looks like a wire cable dragged across it

- Mapped debris field