Drive Firmware Update Instructions

Versions

- Drive Firmware: 01.01.11.00B00

- PAL: 87

- EtherCAT ESI: 0x0010420 V03

Update Instructions

- Upload the current parameter files from each drive (use upload Textual Parameters).

- If any programs exist on the drives, upload copies of those as well

- Update the drive firmware. Power cycle after the update finishes

  - Update can be done via USB or by FoE (if GMAS is available)

  - If using USB, verify your USB COM port in EAS after the power cycle

- Update the drive PAL. Power cycle after the update finishes

  - Update can be done via USB or by FoE (if GMAS is available)

- After powering the drives back up, download the parameter files you saved to each drive then upload a new copy to reflect the new firmware version

  - If there are any errors while downloading, please note and report them to your local Elmo Motion Control office

- Verify 1:1:1 or 1:2:2 timing (see instructions further below)

  - Note: this is only necessary if updating from an firmware earlier than 1.1.7.12B00

- Re-build any drive programs that were uploaded at the beginning of this process

- If using a GMAS or 3rd party EtherCAT device, update the Gold drive ESI file in the ECAT master’s EtherCAT slave XML library and rebuild the ECAT configuration

  - Note: Instructions for updating this file for the GMAS and EAS are below. Only needed for EAS versions 2.01.x.x and earlier; the new ESI is available in EAS 2.2.x.x

Update Drive ESI (XML) file

Note: This should be done for EAS

- Close EAS II

- Delete existing drive ECAT XML file(s) from EAS II slave XML library located in the folders shown below.

  - Delete any existing files with names starting with:

    - “Elmo ECAT DriveGcon”

    - “Elmo ECAT 000103F6”

  - XML library locations:

    - Windows 32-bit: C:\Program Files\Elmo Motion Control\Elmo Application Studio II\EtherCATSlaveLib

    - Windows 64-bit: C:\Program Files (x86)\Elmo Motion Control\Elmo Application Studio II\EtherCATSlaveLib

- Open EAS

- Select Edit EtherCAT Configuration

- Once in the EtherCAT Configurator, right click on the GMAS and select “Start Master (Scan and Download Application)”

- Exit back to system configuration

Verify and change 1:1:1 or 1:2:2 servo timing

- 1:1:1 timing is a new feature which allows the velocity/position/current loops to run every servo cycle (1xTS). Previously the velocity/position loops run every 2xTS and current loop runs every TS.

- The 1:1:1 timing is enabled by default if the drive’s TS parameter is 60 or higher.

- Some Gold drives (Trombone, Bassoon, Drum, Tuba) use TS=100 by default. As a result the drive might switch to 1:1:1 timing from 1:2:2 after the update. This might require re-tuning, but it is also possible to set the timing to 1:2:2 manually.

- To check current timing, after downloading firmware, send the LD command then check WS[61].

  - If WS[61]=0 the timing is 1:2:2

  - If WS[61]=1 the timing is 1:1:1

- To manually set 1:2:2 timing

  - Set XA[5]=2

  - Save to flash (SV)

  - Load from flash (LD)

  - Check WS[61] and verify it is 0. This indicates that the drive is set for 1:2:2 timing