CPF Deployment check list

Gene Massion

Version 1.0 	2014 August 31

Version 1.1	2015 March 30

- PreLoad Software

  - Make sure stuff used in tank testing is changed for deployment

    - Program.cs

      - Set initial state at top of State Machine while loop

      - uBlox.Request fix number of tries and time delay in SurfaceOps

      - Comment out all test compiler directives

    - configFile.cs

      - Set appropriate preMissionDelayTimeout

      - Set appropriate SurfaceOpsTimeout

      - Uncomment SOSetBellowsPosition to correct value

      - Set descendBellowsLimit to bellowsLowerLimit + 3.0;

    - Clear all breakpoints

    - Comment out debug statements

  - Mission Config

    - Make sure last ascend has 0.0 depth

    - usbCable = false

    - Set SBE41 pump time

    - Set SBE41 cpCutoff pressure

  - Set SORecoveryMode as appropriate

  - AnchorPressureMinimum as appropriate

  - Test everything with software on deployment computer

- PreLoad Mechanical

  - Balllast so lifting plate is at waterline with bellows at SOSetBellowsPosition

- PreDeployment Mechanical

  - Remove eye bolts as necessary

  - Remove deploy/debug magnet

  - Remove Optode rubber cover

  - Remove CTD caps

  - Check tension screw cover is tight

  - Check Swagelok fitting is tight

  - Check tapered ring/bellows cover are screwed together tightly

  - Check lifting plate is secured properly

  - Position USBL beacon/GPS tracker are opposite dive weights

- Review InitMission data

  - Check mission is correct

  - Check Optode and CTD settings are correct