CPF-P2 Deployment check list				Date: 10/01/2021

User: Magdalena &  Joseph

Mission: Deep Dive 100m

- Software checks

____Gene/Eric__Clear breakpoints			______Comment out compiler directives

- Mission configuration

_____________Bellows upper/lower limit		______00:01:00___Park hours/minutes

_____50_____ Surface ops bellows position	           ________________Recovery mode BP

_____120_____parkdepth 				________125_____maxPressure

______3______maxProfiles				________________maxDays/Hours

___10:57AM__ Pre mission delay TO			________________Surface ops TO

_____________Descend TimeOut			________________Park TO

_____________Anchor TO				________________Ascend TO

_____________Number of SBD retries		________________ Homer ID

_____________Sample Period

- Pre-Deployment Mechanical

_____Receive SBD message				_____ Receive  GPS tracker message

_____Double check lifting bridle			__Y__ Remove Optode  cover

__Y___Remove CTD caps					__Y__ Remove FLBB cover

__Y___Connector dummy plugs installed		_____Remove GPS tracker off magnet

___________________Number of ballast bars	_____32.75_____Battery bus voltage

_________22.36 %____Humidity/Temp.		_____1225______Int. pressure/Temp.

_____Remove Telonics magnet/Frequency (160.14000MHz)

__Y__Remove ISUS cover

- Deployment

________102.4__________Water Depth m 		______ 10:55______Over the side time

_________10:55______Mission start time ($GO)	_______11:01_______Under Surface time

______36,7329_______Deployment lat. ($GO)		____  121.936_________Deployment long. ($GO)

_________10:58_________AB retract fast start time	____11:02______ Descend time

FORM ON BACK FOR SUBSEQUENT DIVES FROM STARTING PROFILE: