1) Buy DM3CS uSD card connector - Gene

On order

Received

2) Change COM to COMM or something on top level drawing – Gene

Done

3) Check LED reverse voltage with enable jumper removed - Paul

4) Get Paul the details of the mode jumper – Gene

Mode should be pulled high normally and pulled low with a jumper

5) Check bypass caps on processor schematic - Paul

6) Investigate EMMC memory - Eric

7) Remove PTH from motherboard - Paul

8) Investigate system overcurrent fuse size requirements - Ed

9) Find out max current output from both kinds of batteries – Gene

a) The Li-ion packs are 32X Panasonic 18650B cells in a 8S4P arrangement.  The protection circuit trips at “40-60 Amps”.  I’m checking with the vendor to get more details.

b) The primary packs are 2X Electrochem 3B0076 cells in series although we can use one of their other DD cells if we’d like.  We have room for 12X packs and I’ve been thinking we’d arrange them as 4S3P.  Their max continuous current is 3A and they are fused at 4A.  So 12A max.

10) Beef up regen diode - Paul

11) Gene and Ed to talk about regen protection

12) Make sure Jose leaves enough trace to cut/jumper on I2C address lines - Paul

13) Develop an I2C address list for all devices - Gene

14) Look at DC/DC converter input caps - Paul

15) Check stability of input PI filter - Paul

16) Check current limit of DC/DC converters - Paul

17) Add up 5V loads - Gene

18) Look at trip time on Elmo power circuit - Paul

19) Check active state for IRQs (hopefully active low) – Gene

IRQs can be active either low or high also on a falling, leading or both edge(s)

20) Double check connector pinout on motherboard/daughter card - Ed/Paul

21) Check 1.8V on DC clock and see if we can fallback to 3.3 - Ed

22) Maybe add jumper to level translator force on line - Ed

23) Investigate MAX power down functionality - Gene/Brent

In progress

24) Change JP1 and JP2 (and other jumpers) on daughter card to 0 ohm resistor - Ed

25) Use discrete resistors for RN1 on daughter card - Ed

26) Ed and Paul decide about split power and signal ground - Ed/Paul

27) Consider using same TVS - Ed/Paul

28) Consider using Ed's switch on MB - Paul

29) Review implications of using ADIN on energy monitor IC for daughter card channel voltage – Gene

Doesn’t appear to be a problem

30) Send out motherboard layout drawing - Jose

31) Make sure silkscreen has pin numbers on connectors - Jose

32) FLT and IRQ lines are active low - Ed/Paul

See item 19 above