Modifications to Coastal Profiling Float Motherboard to replace I2C buffers

P. McGill

1Apr19

Objective: The six I2C buffers on the original motherboard design were found by Eric Martin to have problems with intermittent latch-up and were replaced by a different IC. Eric also found that the pullup resistors on the daughtercard side needed to be a higher value.

All modifications are based on MB schematic sheet 3 v1.5.0. The schematic with the changes applied is v1.6.0.

- Remove and replace the six TCA4311ADR buffers, U9 through U14, with PCA9511AD.

- Remove and replace the twelve 2.2 kΩ pullup resistors, R38 through R49, with 10 kΩ resistors. The resistors are SMD 0603 5%, but 1% can be substituted.