Battery Input Board (BIB)

Lower Half Electronics (LHE) Primary Battery Board

Initial Board Bring-Up and Testing

Chad Kecy

Revision A

03-29-2023

Battery Input Board (BIB) Testing

Before powering up the LHE I tested out the also new Battery Input Board (BIB). The BIB is used to interface the eight primary batteries to the LHE board. There are eight input connectors, one per battery, and one output connector which supplies battery power to the LHE. The BIB is laid out to provide a quad 2S4P battery connection. Simply put, there are four cells per battery; two in series provides the necessary bus voltage of 25.6Vdc. Four of these packs in parallel provide 104 amp-hours of energy to the CPF. The BIB has blocking diodes and fuses for each of the four packs.

Testing of the BIB entailed verifying the board was laid out and populated correctly. Ohm checks confirmed that the electrical connections were correct. I then applied power to each of the inputs and verified the correct outputs. The BIB was used on all subsequent LHE testing.

Figure One: Block diagram of the BIB connections.

Referring to Figure One, the eight individual 12.8Vdc batteries are connected to the BIB. The traces on the BIB combine 2&4, 6&8, 1&3, and 5&7 into four packs delivering 25.6Vdc (nominal) at 26Ah each.

Issues

No serious issues were found; the board operates as expected. I did notice that the spring activation mechanism is a little wonky on the Phoenix connectors which interface the batteries. When using a small flathead screwdriver to open the wire slot, the orange tab can move sideways a little bit. When installed in the CPF, care should be taken to apply the opening force in the down direction only.

Equipment Used

Extech 382260 Low Voltage Power Supply

Fluke 87 Multimeter

Relevant Battery Input Board (BIB) Files

\\CPF\Electrical Engr\Orcad Projects\Chads Stuff\Pri_Batt_Input_Board

Lower Half Electronics (LHE) Primary Battery Board Testing

There were five boards fabricated with two being populated onsite. The two boards were labeled as SN#01 and SN#02. A bulk of the testing required the rest of the CPF circuit boards to be connected to the LHE. These included the Motherboard, the Power Switch Board, and the Internal Sensor Interface Board. Tera Term on my computer was used to send and receive data.

| Installed | JP1 | Hotel LED |
| --- | --- | --- |
| Installed | JP2 | Pump LED |
| open | JP3 | Sleep Disable |
| Used to turn on | JP4 | Reed Switch On/Off |
| 2-2 | JP5 | Power Override |
| 2-2 | JP6 | Power Override |

Table One: Initial jumper settings

Figure Two: Block diagram of the CPF circuit boards

Equipment Used

Extech 382260 Low Voltage Power Supply (x4)

Fluke 87 Multimeter

Tektronix TDS 2014 Four Channel Digital Oscilloscope

BK Precision 8500 Programmable DC Electronic Load

Prolific PL2303GS USB-Serial Converter

Relevant LHE Primary Battery Board Files

\\CPF\Electrical Engr\Orcad Projects\Chads Stuff\LHE_Pri_Batt_Board

Board Power

For the initial powering of the LHE I used a single Extech power supply and ran the wires through the Battery Input Board. Straight away I noticed the board was not being powered correctly. I eventually discovered the shorting blocks for the i2c addresses were all shorted during population (see Issues below). Upon remedying the problem, the board powered up correctly. All voltages were within specifications, with the exception of pump power due to not having i2c up and running yet.

| Good | TP1 | Hotel Volts (Batt) |
| --- | --- | --- |
|  | TP2 | Hotel Return |
| no i2c yet | TP3 | Pump Volts (Batt) |
|  | TP4 | Pump Return |
| Good | TP5 | Batt5V Spy |
| Good | TP6 | Batt5V Spy |
|  | TP7 | Signal Common |
| Good | TP8 | Mobo 5V Spy |
| Good | TP9 | Mobo 5V Spy |
|  | TP10 | Signal Common |

Table Two: Test point locations for initial power testing

Issues

During initial population, SB1-SB8 were all shorted. This caused Batt5V+ to be shorted to ground and damaged U7. Setting the SB’s to their proper states and replacing U7 brought the Batt5V+ voltage back in compliance. This issue was present on both populated boards, so SN#02 was able to be fixed prior to powering up.

Communication via i2c

To test the i2c communication, the entire CPF board stack needs to be interconnected. See Figure Two above for the connection details. Eric Martin had already written a software interface to the Motherboard and had also written an initial piece of code for the LHE Primary Board.

Initial attempts at i2c communication were unsuccessful. My first troubleshooting step was to verify all of the i2c addresses across all of the interconnected boards. I did have to change the default address on the LHE board as that address was already in use on the Motherboard.

| ADR0 | ADR1 | Hex Addr | Dec Addr | Device |
| --- | --- | --- | --- | --- |
| H | L | CE | 206 |  |
| NC | H | D0 | 208 |  |
| H | H | D2 | 210 | Switch U6 |
| NC | NC | D4 | 212 | LHE U1 |
| NC | L | D6 | 214 | LHE U5 |
| L | H | D8 | 216 | Switch U12 |
| H | NC | DA | 218 |  |
| L | NC | DC | 220 |  |
| L | L | DE | 222 | Mobo U1 |

Table Three: i2c Communication Addresses

At this point, I was able to communicate with the Motherboard via address DE. However, the LHE board was unable to communicate. Using an oscilloscope, I saw no activity on the clock or data lines on the output of U11 (PCA9512). The PCA9512 is an i2c buffer chip which allows hot swapping of daughter boards and provides voltage translation. After subsequent troubleshooting, I found that an incorrect version of the energy monitor IC (LTC2946) had been ordered and installed onto the two LHE boards (see Issues below). Replacing the part fixed the issue and i2c comms worked fine after that.

During my troubleshooting I also noticed that the ADIN pin (pin 13) on U1 was left floating. According to the datasheet, it needs to be tied to some sort of reference voltage in order to make an accurate measurement. I had a voltage divider added to the board in the same way that U5 has. After this mod was installed, I lost i2c comms to the energy monitor chips. After eliminating other possibilities, I had the mod removed and U1 and U5 replaced. Once again, i2c comms returned to normal. Subsequent Hotel Load testing (below) resulted in believable numbers. As of now, the pin is still floating, but the part appears to be working correctly.

Issues

The LTC2946 is an energy monitoring chip; it samples the input voltage and uses a current sense resistor to measure the load current. This part separates the serial data line (SDA) into individual input and output pins (SDAI and SDAO). This allows the user to add in optional external opto transistors for electrical isolation, if desired. A version of the LTC2946 exists which inverts the SDAO line (LTC2946-1). It is pin compatible with the non-inverting version (LTC2946). The inverting part was ordered and installed; the inverted output line locked up all i2c communication to the LHE board. Replacing it with the non-inverting version fixed the issue and all i2c comms started working.

Hotel Load

To test the hotel load, I clipped leads onto TP1 and TP2 and connected those to an electronic dummy load (BK Precision 8500). I varied the load resistance and measured both the analog voltage and current, and also used the i2c bus to read back the energy monitor readings. These values were then plotted and a conversion factor found.

| Hotel Path |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ILoad (nom) | RLoad (omhs) | VLoad (volts) | ILoad (amps) | Volt (Cts) | Amps (Cts) |
| 0 |  |  | 0 | 954 | 5 |
| 0.01 | 2400 | 23.86 | 0.008 | 954 | 7 |
| 0.025 | 960 | 23.84 | 0.023 | 953 | 10 |
| 0.05 | 480 | 23.83 | 0.048 | 953 | 15 |
| 0.1 | 240 | 23.81 | 0.097 | 952 | 25 |
| 0.25 | 96 | 23.77 | 0.246 | 951 | 54 |
| 0.5 | 48 | 23.72 | 0.492 | 950 | 104 |
| 1 | 24 | 23.65 | 0.983 | 948 | 201 |
| 2 | 12 | 23.50 | 1.956 | 946 | 396 |
| 3 | 8 | 23.37 | 2.918 | 944 | 588 |
| 4 | 6 | 23.25 | 3.872 | 942 | 778 |

Table Four: SN#01 Hotel load values

Figure Three: SN#01 Hotel load plot (Measured current vs i2c ADC counts)

| Hotel Path |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ILoad (nom) | RLoad (ohms) | VLoad (volts) | ILoad (amps) | Volt (Cts) | Amps (Cts) |
| 0 |  |  | 0 | 954 | 5 |
| 0.01 | 2400 | 23.87 | 0.010 | 954 | 7 |
| 0.025 | 960 | 23.85 | 0.024 | 954 | 10 |
| 0.05 | 480 | 23.84 | 0.049 | 953 | 15 |
| 0.1 | 240 | 23.82 | 0.098 | 953 | 25 |
| 0.25 | 96 | 23.78 | 0.247 | 951 | 54 |
| 0.5 | 48 | 23.73 | 0.493 | 950 | 104 |
| 1 | 24 | 23.65 | 0.984 | 949 | 202 |
| 2 | 12 | 23.50 | 1.956 | 946 | 396 |
| 3 | 8 | 23.36 | 2.918 | 944 | 589 |
| 4 | 6 | 23.23 | 3.869 | 941 | 779 |

Table Five: SN#02 Hotel load values

Figure Four: SN#02 Hotel load plot (Measured current vs i2c ADC counts)

In addition to the load current measurements, I also tested the undervoltage warning and shutdown functionality. An undervoltage warning is flagged when the battery voltage reaches nominally 18Vdc; the system will experience a hardware shutdown when the battery voltage reaches nominally 16Vdc.

|  | SN# 01 | SN# 02 |
| --- | --- | --- |
| Signal | (volts) | (volts) |
| N_UV_WARN | 18.09 | 17.94 |
| Shutdown | 16.11 | 16.12 |

Table Six: Undervoltage measurements

Pump Load

To test the pump power path, I used a test cable to connect J4 to the electronic dummy load. I varied the load resistance and measured both the analog voltage and current, and also used the i2c bus to read back the energy monitor readings. These values were then plotted and a conversion factor found.

| Pump Path |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ILoad (nom) | RLoad (omhs) | VLoad (volts) | ILoad (amps) | Volt (Cts) | Amps (Cts) |
| 0 | None | 0.34 | 0 | 15 | 0 |
| 0 | None | 23.88 | 0 | 954 | 0 |
| 0.5 | 48 | 23.78 | 0.493 | 950 | 59 |
| 1 | 24 | 23.74 | 0.987 | 949 | 117 |
| 2 | 12 | 23.67 | 1.970 | 947 | 234 |
| 3 | 8 | 23.61 | 2.950 | 945 | 350 |
| 4 | 6 | 23.56 | 3.925 | 944 | 466 |
| 6 | 4 | 23.45 | 5.863 | 941 | 696 |
| 8 | 3 | 23.35 | 7.785 | 938 | 924 |
| 10 | 2.4 | 23.26 | 9.690 | 936 | 1150 |
| 12 | 2 | 23.16 | 11.578 | 933 | 1375 |
| 13 | 1.85 | 23.10 | 12.484 | 931 | 1483 |

Table Seven: SN#01 Pump power path load values

Figure Five: SN#01 Pump power plot (Measured current vs i2c ADC counts)

| Pump Path |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ILoad (nom) | RLoad (ohms) | VLoad (volts) | ILoad (amps) | Volt (Cts) | Amps (Cts) |
| 0 | None | 0.36 | 0 | 15 | 0 |
| 0 | None | 23.87 | 0 | 954 | 1 |
| 0.5 | 48 | 23.77 | 0.492 | 950 | 60 |
| 1 | 24 | 23.73 | 0.985 | 949 | 118 |
| 2 | 12 | 23.66 | 1.968 | 947 | 236 |
| 3 | 8 | 23.61 | 2.949 | 945 | 353 |
| 4 | 6 | 23.56 | 3.925 | 944 | 469 |
| 6 | 4 | 23.45 | 5.863 | 941 | 700 |
| 8 | 3 | 23.36 | 7.785 | 938 | 928 |
| 10 | 2.4 | 23.26 | 9.690 | 935 | 1156 |
| 12 | 2 | 23.16 | 11.579 | 932 | 1382 |
| 13 | 1.85 | 23.10 | 12.484 | 931 | 1490 |

Table Eight: SN#02 Pump power path load values

Figure Six: SN#02 Pump power plot (Measured current vs i2c ADC counts)

Sleep Circuit

In order to save battery energy, the LHE board can be put into sleep mode from 2^(n-1) minutes at a time where n is an integer from 1 to 7. This corresponds to fixed sleep times of 1, 2, 4, 8, 16, 32, or 64 minutes. The heart of the sleep circuit is a MC14536 timer chip. A combination of an analog (RC) frequency clock and digital inputs determine how often the output signal changes state, thus initiating a sleep or non-sleep cycle. I ran through each of the seven fixed sleep timers and measured the actual sleep time.

|  |  |  | (minutes) | (minutes) | (seconds) | (seconds) | (seconds) | (minutes) | (minutes) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Command | n | n-1 | 2^(n-1) | Meas Time | Meas Time | Nom Time | Delta | Expected | Measured |
|  | 0 |  |  |  |  |  |  |  |  |
| Sleep 1 | 1 | 0 | 1 | 1:08 | 68 | 60 | 8 | 1 | 1.13 |
| Sleep 2 | 2 | 1 | 2 | 2:16 | 136 | 120 | 16 | 2 | 2.27 |
| Sleep 3 | 3 | 2 | 4 | 4:31 | 271 | 240 | 31 | 4 | 4.52 |
| Sleep 4 | 4 | 3 | 8 | 9:01 | 541 | 480 | 61 | 8 | 9.02 |
| Sleep 5 | 5 | 4 | 16 | 18:00 | 1080 | 960 | 120 | 16 | 18.00 |
| Sleep 6 | 6 | 5 | 32 | 35:57 | 2157 | 1920 | 237 | 32 | 35.95 |
| Sleep 7 | 7 | 6 | 64 | 71:51 | 4311 | 3840 | 471 | 64 | 71.85 |

Table Nine: SN#01 Sleep circuit measurements

|  |  |  | (minutes) | (minutes) | (seconds) | (seconds) | (seconds) | (minutes) | (minutes) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Command | n | n-1 | 2^(n-1) | Meas Time | Meas Time | Nom Time | Delta | Expected | Measured |
|  | 0 |  |  |  |  |  |  |  |  |
| Sleep 1 | 1 | 0 | 1 | 1:09 | 69 | 60 | 9 | 1 | 1.15 |
| Sleep 2 | 2 | 1 | 2 | 2:17 | 137 | 120 | 17 | 2 | 2.28 |
| Sleep 3 | 3 | 2 | 4 | 4:34 | 274 | 240 | 34 | 4 | 4.57 |
| Sleep 4 | 4 | 3 | 8 | 9:06 | 546 | 480 | 66 | 8 | 9.10 |
| Sleep 5 | 5 | 4 | 16 | 18:09 | 1089 | 960 | 129 | 16 | 18.15 |
| Sleep 6 | 6 | 5 | 32 | 36:16 | 2176 | 1920 | 256 | 32 | 36.27 |
| Sleep 7 | 7 | 6 | 64 | 72:28 | 4348 | 3840 | 508 | 64 | 72.47 |

Table Ten: SN#02 Sleep circuit measurements

Issues

Both boards ran slow on the sleep timer by about eight seconds per minute. At the longest sleep period of 64 minutes, this results in an actual sleep time of 72 minutes. I am currently leaving the sleep circuit as is. If needed, the timing could be modified by changing the RC values for the MC4536 timer.

Sleep Power Consumption

While the LHE board was in sleep mode, I measured the Batt5V+ current draw by monitoring TP5 and TP6. I measured 4.14mV. The sense resistor measured 0.98 ohms, which results in a sleep current of 4.22mA. This is significantly higher than the design goal of ~100uA. It appears that the bulk of this excess sleep current is originating from the two LTC2946 energy monitor chips. According to the datasheet, they need to be put into a shutdown mode via an i2c command before the LHE board is put to sleep. I verified from Eric Martin that this is not part of the current code. This can be retested after Eric has a chance to modify the code.

Conclusion

Overall, the initial bring-up of the LHE board was very successful, with the exception of a few minor problems.

- Both energy monitor chips (U1 and U5) need to be placed in shutdown mode prior to the LHE board going into sleep mode. This is done via a software command over the i2c bus.

- Sleep timer is slow by eight seconds per minute. This delay is linear with increasing sleep times.

- The largest sleep period is ~1hr (72 minutes with the accumulated delays)

- Reminder to remove the LED jumpers prior to deployment

- Document all solder bridge connections for i2c addresses

- The Pumpmon command appears to be reading the Hotel load (maybe from the Mobo?)