CPF Internal Sensors Board Engineering Change Order List

ECO A: PTH Sensor Power Up/Logic Line

- Remove Q7

- Jumper Q7-3 pad tp Q7-6 pad

- Remove R31, R32, R33

ECO B: Remove bus contention signals

- Cut the trace coming from J9-A2 [rear side]

- Cut the trace coming from J9-B2 [rear side]

ECO C: Reroute signals from ECO B

- Remove U9

- Attach a wire from U9-2 pad to U4-7

- Attach a wire from J9-B9 to U5-9