LHE CCA Design Review

Feb 22, 2019

Gene Massion

Notes:

- Double check system fuse rating vs 8 amp requirement vs 12.5 amp circuit breaker vs Elmo over current limit.

- Make sure the fuse is the last thing that trips.

- Decide whether to add a connector or even the regen circuitry to allow regen circuitry to be added if we switch to a bigger pump.

- Make sure the LHE ground scheme is consistent with the rest of the float grounding details.

- Check if dumb I2C switches are a safer choice than electronic switch.

- Check into battery switching to master mode and squawk (per Eric’s comment).

- Think about having 2 I2C switches controlled by 2 devices (the charger and the Mobo).

- Maybe add an I2C buffer to the LHE to define the pullup voltage on the LHE.

- Make sure not to switch the batteries to fast in order to prevent the blown fuse issue the LRAUV group saw.

- Think about 3A and 10A (and all other fuses).  Maybe use lower amp rated fuses during development and replace with higher rated fuses for deployment.

- Might be worth calculating the energy loss in the ideal diode and ID controller vs non-ideal diode.

- Modify DIO SOW and Top level diagrams.

- Make sure Jose has the right board outline and decide if we need a multi-part board.

Added 2019-Feb-25:

- Since batt pack will now be a discrete assembly, add jumpers or switches to manually power the pack up or down, and hold it in that state.

Added 2019-Feb-26:

- Add coil suppression diodes on deadface relays

- Take 28V feed for Batt5V from TWO sticks for redundancy, diode-OR.

- Add ripple cap(s) to motor drive port