CPF Switch Board

Configuration Template

Rev A	20 Oct 16	ecm

Board S/N:	3

Schematic Release:  	E

PWB Rev:		A

Application:		BOG CPF SN2

Config by:		BDJ

Date:			11JAN17

1.8/3.3V Config

Default shown is 1.8V operation.

Section				1.8V			3.3V

Oscillator 		R2  	0 ohm		R1   	open

UART A (U2) 		R6  	0 ohm		R5   	open

UART B (U7) 		R28  	0 ohm		R27   	open

UART C (U12) 		R50  	0 ohm		R49   	open

Comm A Config - Iridium

Component			Native Serial		UART			CTD

U2 (XR20M1280)		remove		install			remove

U3 (Max3244)			install			install			remove

R7				remove		install			remove

R8				install			remove		remove

R9				install			remove		remove

R10				remove		install			remove

R4 (CTD pullup)		remove		remove		install

R17 (Max3244 manual)	install			install

R18 (Max3244 Auto)		remove		remove

Address for UART A:

SB1							open

SB2							open

SB3							open

SB4							open

SB5							open

SB6							open

SB7							open

SB8							open

Pwr A Config

Voltage Select – Both jumpers MUST match

12V			5V

JP1 (input select)		1-2			3-2

JP2 (output select)		1-2			3-2

R23 (voltage scaling)		453k			165k

Amperage select		1.25A			2.5A

Q1				FD8896		IRF7440

R21				0.04Ω			0.02Ω

Energy Monitor

U6				Remove

Address 0xD4 for Energy Monitor A is shown:

SB9				open

SB10				open

SB11				open

SB12				open

Comm B Config FLBB

Component			Native Serial		UART

U7 (XR20M1280)		remove		install

U8 (Max3226)			install			install

R29				remove		install

R30				remove		install

R31				install			remove

R32				install			remove

R33 (Max3244 manual)	install			install

R35 (Max3244 Auto)		remove		remove

Address for UART B is shown:

SB13							open

SB14							open

SB15							open

SB16							open

SB17							open

SB18							open

SB19							open

SB20							open

Pwr B Config

Voltage Select – Both jumpers MUST match.  Default 12V shown.

12V			5V

JP3 (input select)		1-2			3-2

JP4 (output select)		1-2			3-2

R45 (voltage scaling)		453k			165k

Amperage select		1.25A			2.5A

Q2				FD8896		IRF7440

R43				0.04Ω			0.02Ω

Energy Monitor

U10				Remove

Address Energy Monitor B is shown:

SB21				open

SB22				open

SB23				open

SB24				open

Comm C Config OPTODE

Component			Native Serial		UART

U12 (XR20M1280)		remove		install

U13 (Max3226)		install			install

R51				remove		install

R52				remove		install

R53				install			remove

R54				install			remove

R55 (Max3244 manual)	install			install

R57 (Max3244 Auto)		remove		remove

Address for UART C is shown:

SB25							open

SB26							open

SB27							open

SB28							open

SB29							open

SB30							open

SB31							open

SB32							open

Pwr C Config

Voltage Select – Both jumpers MUST match.  Default 12V shown.

12V			5V

JP5 (input select)		1-2			3-2

JP6 (output select)		1-2			3-2

R68 (voltage scaling)		453k			165k

Amperage select		1.25A			2.5A

Q3				FD8896		IRF7440

R65				0.04Ω			0.02Ω

Address for Energy Monitor C is shown:

SB33				open

SB34				open

SB35				open

SB36				open