7/22/14 Dive 1 Deployed 09:49

//Ascend table

ascendTable[0].depth = 6;

ascendTable[0].PV = 0.05;

ascendTable[0].parkTime = new TimeSpan(0, 0, 2, 00);         //days, hours, minutes, seconds

ascendTable[1].depth = 3;

ascendTable[1].PV = 0.02;

ascendTable[1].parkTime = new TimeSpan(0, 0, 2, 0);

// The last entry in the Ascend table must have a depth of 0.0

// When the mission hits the last entry in the ascend table, it goes to the surface at this PV rate

// The park time value don't matter,

ascendTable[2].depth = 0.0;

ascendTable[2].PV = 0.03;

ascendTable[2].parkTime = new TimeSpan(0, 0, 0, 0);

descendTable[0].depth = 7.0;

descendTable[0].PV = -0.1;

descendTable[0].parkTime = new TimeSpan(0, 0, 3, 00);

//nudge control bands

public const double OB = 2;    //"outer band" in meters

public const double IB = 0.5;  //"inner band" in meters

public const double approachVel = 0.01;   // absolute approach velocity in m/s

String went taught, indicating float overshot 7m target depth.

9:58 float is oscillating at bottom of line (in ascent?)

10:02 float still oscillating at bottom of line

10:08 float no longer oscillating, still at bottom of line

10:10 appears to be trying to ascend, oscillating again.

10:15 aborting mission and pulling to surface

Notes: D gain was much higher (50) than in previous successful velocity control events (10). Will change all gains back to previous (P=70, D=10) and retry!

7/22/14 Dive 2 Deployed 10:22

//Ascend table

ascendTable[0].depth = 6;

ascendTable[0].PV = 0.05;

ascendTable[0].parkTime = new TimeSpan(0, 0, 2, 00);         //days, hours, minutes, seconds

ascendTable[1].depth = 3;

ascendTable[1].PV = 0.02;

ascendTable[1].parkTime = new TimeSpan(0, 0, 2, 0);

// The last entry in the Ascend table must have a depth of 0.0

// When the mission hits the last entry in the ascend table, it goes to the surface at this PV rate

// The park time value don't matter,

ascendTable[2].depth = 0.0;

ascendTable[2].PV = 0.03;

ascendTable[2].parkTime = new TimeSpan(0, 0, 0, 0);

descendTable[0].depth = 7.0;

descendTable[0].PV = -0.1;

descendTable[0].parkTime = new TimeSpan(0, 0, 3, 00);

//nudge control bands

public const double OB = 2;    //"outer band" in meters

public const double IB = 0.5;  //"inner band" in meters

public const double approachVel = 0.01;   // absolute approach velocity in m/s

public const double KPlatVel = 70.0,                        //Constant for P term of PID

TiPlatVel = 50000.0,

TdPlatVel = 10.0,                        // Time constant for D term of PID

TfPlatVel = 2.0,                        // Derivative filter time constant

TtPlatVel = 100.0,                      // Anti-windup time constant

BPlatVel = 1.0,                         // Setpoint weight

uLowPlatVel = -3000.0 / 60.0,           // minimum possible output (RPS)

uHighPlatVel = 3000.0 / 60.0,           // maximum possible output (RPS)

hPlatVel = DGHSamplePeriod / 1000.0;  // Sample period (seconds)

10:28 float stopped before reaching bottom of line. Slowly drifting upwards after stopping

10:29 float seems to have stopped

10:30 float hasn’t moved…seems to be holding position

10:31 float descending very slowly

10:36 float descending slowly (got out of zero-velocity band?)

10:39 float ascending slowly

10:41 descending (oscillating?)

10:42 ascending

10:44 descending (def. oscillating…and should have reached higher set point by now, I think oscillation is preventing it from reaching higher up deadband, which would in turn prevent system from incrementing to next depthTableNum…but system should timeout after..1h20m according to current settings.

10:47 aborting and pulling to surface

Looking at log file, system never enters velocity deadband, bounces between -0.01 and 0.01 m/s

7/22/14 Dive 3 Deployed 11:18

Same settings as Dive 2, but with sign in if statement changed that hopefully puts float into deadband appropriately (code improperly written

11:26 stopped before reaching bottom of line, has slight positive velocity

11:29, seems to have reached 0 velocity and not moving

11:30 sunk to bottom of line

11:33 still at bottom of line…damn

11:34 aborting and pulling to surface to figure out problem

Upon pulling float up, seems line was not taught on bottom…did platform indeed reach 0 velocity near bottom of line? Perhaps after rebounding due to line stretch?

Logfile ended abruptly at 11:25:26, but platform reached zero velocity and stayed in band @ 7m…WTF?

7/22/14 Dive 4 Deployed 13:05

Same PID gains as above

Same depth bands as above

ascendTable[0].depth = 1;

ascendTable[0].PV = 0.05;

ascendTable[0].parkTime = new TimeSpan(0, 0, 2, 00);         //days, hours, minutes, seconds

ascendTable[1].depth = 0;

ascendTable[1].PV = 0.02;

ascendTable[1].parkTime = new TimeSpan(0, 0, 2, 0);

// The last entry in the Ascend table must have a depth of 0.0

// When the mission hits the last entry in the ascend table, it goes to the surface at this PV rate

// The park time value don't matter,

ascendTable[2].depth = 0.0;

ascendTable[2].PV = 0.03;

ascendTable[2].parkTime = new TimeSpan(0, 0, 0, 0);

descendTable[0].depth = 2.0;

descendTable[0].PV = -0.05;

descendTable[0].parkTime = new TimeSpan(0, 0, 3, 00);

7/25/14 Dive 1 Deploy 12:

ascendTable[0].depth = 4;

ascendTable[0].PV = 0.05;

ascendTable[0].parkTime = new TimeSpan(0, 0, 5, 0);         //days, hours, minutes, seconds

ascendTable[1].depth = 0.0;

ascendTable[1].PV = 0.05;

ascendTable[1].parkTime = new TimeSpan(0, 0, 0, 0);

descendTable[0].depth = 7;

descendTable[0].PV = -0.05;

descendTable[0].parkTime = new TimeSpan(0, 0, 5, 0);

//nudge control bands

public const double OB = 2;    //"outer band" in meters

public const double IB = 1.0;  //"inner band" in meters

public const double approachVel = 0.01;

7/25/14 Dive  2

ascendTable[0].depth = 4;

ascendTable[0].PV = 0.05;

ascendTable[0].parkTime = new TimeSpan(0, 0, 5, 0);         //days, hours, minutes, seconds

ascendTable[1].depth = 0.0;

ascendTable[1].PV = 0.05;

ascendTable[1].parkTime = new TimeSpan(0, 0, 0, 0);

descendTable[0].depth = 7;

descendTable[0].PV = -0.05;

descendTable[0].parkTime = new TimeSpan(0, 0, 5, 0);

//nudge control bands

public const double OB = 1;    //"outer band" in meters

public const double IB = 0.2;  //"inner band" in meters

public const double approachVel = 0.01;