CPF FEA Analysis

- Pipe

  - 2D simplification

  - 3D stress

  - Buckling

- Top Flange

  - 2D simplification

  - 3D stress

- Housing Base

  - 2D simplification

  - 3D stress

- Assembly

  - 2D simplification

  - 3D stress

  - 3D quarter model

    - Pick the “worst case” quarter with respect to holes

  - Buckling

- General setup

  - Check material(s)

  - Properties

    - Solver

      - Automatic

      - Use soft springs to stabilize models if necessary

      - Use inertial relief if necessary

  - Connections

    - Globally bonded

    - Make sure to add no penetration where necessary in assemblies

  - Loads

    - 750 psi pressure

    - Load the o-ring feature surfaces realistically

  - Add split lines where necessary for o-rings

  - Use the default mesh to start

    - Refine the mesh as necessary

    - Make sure there are at least 2 elements in every direction

- Figure out how to use the report generator

- Questions

  - What is the difference between soft springs and inertial relief

  - How does FEA buckling compare with Roark and other equations

    - FEA an ideal cylinder can compare to Roark

    - If the end caps hold the ends round does that help?

  - Is globally bonded the right starting condition?

  - How does moving the restraint on a split line change the resutls