|  | Simulation of FFS CPF Date: Friday, March 27, 2015 Designer: Analysis was performed by Farley F. Shane Study name: 260315 CPF Buckling 1 Analysis type: Buckling. Table of Contents Description 1 Model information 2 Study Properties 4 Units 4 Material Properties 5 Loads and Fixtures 6 Connector Definitions 6 Contact Information 7 Mesh Information 9 Sensor Details 11 Study Results 12 Conclusion 13. |
| --- | --- |
| # Description Buckling Analysis for the Coastal Profiling Float 26March2015 |  |

# Model information Model name: FFS CPF Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Cut-Revolve2; Solid Body; Mass:16.7098 lb Volume:171.306 in^3 Density:0.0975437 lb/in^3 Weight:16.6985 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS PipeSch40x8.0-cpfV2.SLDPRT Mar 26 16:01:21 2015. Revolve1; Solid Body; Mass:3.55334 lb Volume:36.4282 in^3 Density:0.0975437 lb/in^3 Weight:3.55093 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS housingBase-cpfV2.SLDPRT Mar 16 15:23:24 2015. Split Line5; Solid Body; Mass:3.8916 lb Volume:39.896 in^3 Density:0.0975437 lb/in^3 Weight:3.88897 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS topFlange-cpfV2.SLDPRT Mar 18 14:21:51 2015. Comments: This analysis was done on simplified End Caps and the housing

# Study Properties Study name; 260315 CPF Buckling 1. Analysis type; Buckling. Mesh type; Solid Mesh. Number of modes; 1. Solver type; Direct sparse solver. Incompatible bonding options; Automatic. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Soft Spring:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Mass density:; 0.0975437 lb/in^3. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Cut-Revolve2)(FFS PipeSch40x8.0-cpfV2-1), SolidBody 1(Revolve1)(FFS housingBase-cpfV2-1), SolidBody 1(Split Line5)(FFS topFlange-cpfV2-1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 edge(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Pressure-1; Entities:; 10 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information Contact; Contact Image; Contact Properties. Contact Set-1; Type:; Bonded contact pair. Entites:; 2 face(s).. Contact Set-2; Type:; Bonded contact pair. Entites:; 2 face(s).. Contact Set-3; Type:; Bonded contact pair. Entites:; 2 face(s).. Global Contact; Type:; Bonded. Components:; 1 component(s). Options:; Compatible mesh..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.25 in. Minimum element size; 0.05 in. Mesh Quality; High. Remesh failed parts with incompatible mesh; On. ## Mesh Information - Details Total Nodes; 361253. Total Elements; 205204. Maximum Aspect Ratio; 258.09. % of elements with Aspect Ratio < 3; 90.5. % of elements with Aspect Ratio > 10; 2.29. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:34. Computer name:; ENIGMA. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 3 face(s). Units:; in. Size:; 0.25. Ratio:; 1.5..

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Amplitude1; AMPRES: Resultant Amplitude Plot for Mode Shape: 1(Load Factor = 1.29431); 0 Node: 185560; 0.000252207 Node: 163138. FFS CPF-260315 CPF Buckling 1-Amplitude-Amplitude1. Name; Type. Displacement1; AMPRES: Resultant Amplitude. FFS CPF-260315 CPF Buckling 1-Displacement-Displacement1. Mode List Mode Number; Load Factor. 1; 1.2943.

# Conclusion