|  | Simulation of FFS CPF Date: Tuesday, March 31, 2015 Designer: FEA performed by Farley F. Shane Study name: 31Mar2015 CPF Assy Buckling 2 Analysis type: Buckling. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh Information 7 Sensor Details 9 Study Results 10 Conclusion 12. |
| --- | --- |
| # Description Coastal Profiling Float Housing Assembly Coarse Mesh |  |

# Assumptions

# Model Information Model name: FFS CPF Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror1; Solid Body; Mass:16.655 lb Volume:170.744 in^3 Density:0.0975437 lb/in^3 Weight:16.6437 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS PipeSch40x8.0-cpfV2.SLDPRT Mar 27 14:40:27 2015. 17/64 (0.26563) Diameter Hole1; Solid Body; Mass:3.457 lb Volume:35.4405 in^3 Density:0.0975437 lb/in^3 Weight:3.45466 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS housingBase-cpfV2.SLDPRT Mar 27 14:40:27 2015. CBORE for 7/16 Hex Head Bolt1; Solid Body; Mass:3.53825 lb Volume:36.2735 in^3 Density:0.0975437 lb/in^3 Weight:3.53585 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS topFlange-cpfV2.SLDPRT Mar 30 17:30:45 2015.

# Study Properties Study name; 31Mar2015 CPF Assy Buckling 2. Analysis type; Buckling. Mesh type; Solid Mesh. Number of modes; 1. Solver type; Direct sparse solver. Incompatible bonding options; Automatic. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Soft Spring:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Mass density:; 0.0975437 lb/in^3. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Mirror1)(FFS PipeSch40x8.0-cpfV2-1), SolidBody 1(17/64 (0.26563) Diameter Hole1)(FFS housingBase-cpfV2-1), SolidBody 1(CBORE for 7/16 Hex Head Bolt1)(FFS topFlange-cpfV2-1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Reference Geometry-1; Entities:; 6 face(s). Reference:; Face< 1 >. Type:; Use reference geometry. Translation:; ---, ---, 0. Units:; in.. Load name; Load Image; Load Details. Pressure-1; Entities:; 20 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information Contact; Contact Image; Contact Properties. Contact Set-1; Type:; Bonded contact pair. Entites:; 2 face(s).. Contact Set-2; Type:; Bonded contact pair. Entites:; 2 face(s).. Global Contact; Type:; Bonded. Components:; 1 component(s). Options:; Compatible mesh..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 1.81054 in. Minimum element size; 0.362108 in. Mesh Quality; High. Remesh failed parts with incompatible mesh; On. ## Mesh Information - Details Total Nodes; 190954. Total Elements; 112720. Maximum Aspect Ratio; 499.81. % of elements with Aspect Ratio < 3; 77. % of elements with Aspect Ratio > 10; 4.59. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:18. Computer name:; ENIGMA. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 2 face(s). Units:; in. Size:; 0.452635. Ratio:; 1.5.. Control-2; Entities:; 1 face(s). Units:; in. Size:; 0.25. Ratio:; 1.5.. Control-3; Entities:; 10 face(s). Units:; in. Size:; 0.05. Ratio:; 1.5..

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Amplitude1; AMPRES: Resultant Amplitude Plot for Mode Shape: 1(Load Factor = 1.87451e-010); 1.3987e-006 Node: 44887; 0.000237284 Node: 86769. FFS CPF-31Mar2015 CPF Assy Buckling 2-Amplitude-Amplitude1. Name; Type. Displacement1; Deformed Shape. FFS CPF-31Mar2015 CPF Assy Buckling 2-Displacement-Displacement1. Mode List Mode Number; Load Factor. 1; 1.8745e-010. Coastal Profiling Float Housing Assembly.

# Conclusion