|  | Simulation of FFS PipeSch40x8.0-cpfV2 Date: Tuesday, May 12, 2015 Designer: FEA Run by Farley F. Shane Study name: 12May2015 7 Ring - 3in - 8-375 Dia Buckling Analysis type: Buckling. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh information 7 Sensor Details 8 Study Results 9 Conclusion 10. |
| --- | --- |
| # Description 12May20157 RIng _ 8.375 Diameter Buckling |  |

# Assumptions

# Model Information Model name: FFS PipeSch40x8.0-cpfV2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror1; Solid Body; Mass:8.56675 kg Volume:0.00317287 m^3 Density:2700 kg/m^3 Weight:83.9541 N; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS PipeSch40x8.0-cpfV2.SLDPRT May 12 13:58:25 2015.

# Study Properties Study name; 12May2015 7 Ring - 3in - 8-375 Dia Buckling. Analysis type; Buckling. Mesh type; Solid Mesh. Number of modes; 1. Solver type; Direct sparse solver. Incompatible bonding options; Automatic. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Soft Spring:; On. Result folder; SOLIDWORKS document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75e+008 N/m^2. Tensile strength:; 3.1e+008 N/m^2. Mass density:; 2700 kg/m^3. Elastic modulus:; 6.9e+010 N/m^2. Poisson's ratio:; 0.33. Thermal expansion coefficient:; 2.4e-005 /Kelvin.; SolidBody 1(Mirror1)(FFS PipeSch40x8.0-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. On Cylindrical Faces-1; Entities:; 2 face(s). Type:; On Cylindrical Faces. Translation:; -0.002, ---, ---. Units:; in.. On Flat Faces-1; Entities:; 1 face(s). Type:; On Flat Faces. Translation:; 0, ---, ---. Units:; in.. Load name; Load Image; Load Details. Force-1; Entities:; 2 face(s). Type:; Apply normal force. Value:; 23289 lbf.. Force-2; Entities:; 2 face(s). Type:; Apply normal force. Value:; 20531 lbf.. Pressure-1; Entities:; 33 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.856957 in. Minimum element size; 0.171391 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 47408. Total Elements; 23865. Maximum Aspect Ratio; 660.87. % of elements with Aspect Ratio < 3; 26.4. % of elements with Aspect Ratio > 10; 24.2. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:07. Computer name:; ENIGMA.

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Amplitude1; AMPRES: Resultant Amplitude Plot for Mode Shape: 1(Load Factor = 1.37854); 5.21647e-008 Node: 8220; 0.000245765 Node: 18428. FFS PipeSch40x8.0-cpfV2-12May2015 7 Ring - 3in - 8-375 Dia Buckling-Amplitude-Amplitude1. Mode List Mode Number; Load Factor. 1; 1.3785.

# Conclusion