|  | Simulation of FFS PipeSch40x8.0-cpfV2 Date: Monday, April 13, 2015 Designer: FEA Run by Farley F. Shane Study name: 13Apr2015 Buckling 1 Designed Housing Analysis type: Buckling. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 7 Contact Information 7 Mesh Information 8 Sensor Details 9 Study Results 10 Conclusion 11. |
| --- | --- |
| # Description 13Apr2015 CPF Housing Buckling 1 |  |

# Assumptions

# Model Information Model name: FFS PipeSch40x8.0-cpfV2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror1; Solid Body; Mass:16.655 lb Volume:170.744 in^3 Density:0.0975437 lb/in^3 Weight:16.6437 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS PipeSch40x8.0-cpfV2.SLDPRT Apr 10 14:15:51 2015.

# Study Properties Study name; 13Apr2015 Buckling 1 Designed Housing. Analysis type; Buckling. Mesh type; Solid Mesh. Number of modes; 1. Solver type; Direct sparse solver. Incompatible bonding options; Automatic. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Soft Spring:; On. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Mass density:; 0.0975437 lb/in^3. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Mirror1)(FFS PipeSch40x8.0-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Reference Geometry-1; Entities:; 1 edge(s). Reference:; Face< 1 >. Type:; Use reference geometry. Translation:; ---, ---, 0. Units:; in.. On Cylindrical Faces-1; Entities:; 1 face(s). Type:; On Cylindrical Faces. Translation:; 0, ---, ---. Units:; in.. Load name; Load Image; Load Details. Pressure-1; Entities:; 5 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg.. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 39810 lbf.. Force-2; Entities:; 1 face(s). Type:; Apply normal force. Value:; 45157 lbf.. Force-3; Entities:; 1 face(s). Type:; Apply normal force. Value:; 32531 lbf.. Force-4; Entities:; 1 face(s). Type:; Apply normal force. Value:; 36901 lbf..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.8 in. Minimum element size; 0.16 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 49345. Total Elements; 24877. Maximum Aspect Ratio; 445.44. % of elements with Aspect Ratio < 3; 28. % of elements with Aspect Ratio > 10; 24.4. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:08. Computer name:; ENIGMA.

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Amplitude1; AMPRES: Resultant Amplitude Plot for Mode Shape: 1(Load Factor = 1.13911e-006); 0.000310667 Node: 16752; 0.00033574 Node: 9353. FFS PipeSch40x8.0-cpfV2-13Apr2015 Buckling 1 Designed Housing-Amplitude-Amplitude1. Name; Type; Min; Max. Amplitude2; AMPRES: Resultant Amplitude Plot for Mode Shape: 1(Load Factor = 1.13911e-006); 0.000310667 Node: 16752; 0.00033574 Node: 9353. FFS PipeSch40x8.0-cpfV2-13Apr2015 Buckling 1 Designed Housing-Amplitude-Amplitude2. Mode List Mode Number; Load Factor. 1; 1.1391e-006.

# Conclusion