|  | Simulation of FFS PipeSch40x8.0-cpfV2 Date: Tuesday, April 14, 2015 Designer: FEA Run by Farley F. Shane Study name: 14Apr2015 Static CPF Hsg 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 7 Contact Information 7 Mesh Information 8 Sensor Details 9 Resultant Forces 10 Beams 10 Study Results 11 Conclusion 15. |
| --- | --- |
| # Description 14Apr2015 CPF Static Analysis for Stock Dia. Housing |  |

# Assumptions

# Model Information Model name: FFS PipeSch40x8.0-cpfV2 Current Configuration: Stock Housing w-O-Rings. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror1; Solid Body; Mass:26.8276 lb Volume:275.032 in^3 Density:0.0975437 lb/in^3 Weight:26.8094 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS PipeSch40x8.0-cpfV2.SLDPRT Apr 10 14:15:51 2015.

# Study Properties Study name; 14Apr2015 Static CPF Hsg 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Mass density:; 0.0975437 lb/in^3. Shear modulus:; 3.77098e+006 psi. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Mirror1)(FFS PipeSch40x8.0-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Reference Geometry-1; Entities:; 1 edge(s). Reference:; Face< 1 >. Type:; Use reference geometry. Translation:; ---, ---, 0. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); -11.8361; 2.46741; -8.95915; 15.0482. Reaction Moment(lbf.in); 0; 0; 0; 0.. On Cylindrical Faces-1; Entities:; 1 face(s). Type:; On Cylindrical Faces. Translation:; 0, ---, ---. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); -20.0179; -17.9923; 12.7369; 29.7769. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 2 face(s). Type:; Apply normal force. Value:; 23278 lbf.. Force-2; Entities:; 2 face(s). Type:; Apply normal force. Value:; 20522 lbf..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.5 in. Minimum element size; 0.1 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 103713. Total Elements; 53593. Maximum Aspect Ratio; 376.31. % of elements with Aspect Ratio < 3; 66.2. % of elements with Aspect Ratio > 10; 20.6. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:16. Computer name:; ENIGMA.

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -20.0179; -17.9923; 12.7369; 29.7769. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 516.358 psi Node: 15102; 39991.1 psi Node: 19364. FFS PipeSch40x8.0-cpfV2-14Apr2015 Static CPF Hsg 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.00027744 in Node: 59177; 0.0154506 in Node: 614. FFS PipeSch40x8.0-cpfV2-14Apr2015 Static CPF Hsg 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.57503e-005 Element: 49718; 0.00199588 Element: 50107. FFS PipeSch40x8.0-cpfV2-14Apr2015 Static CPF Hsg 1-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 0.997357 Node: 19364; 10 Node: 1060. FFS PipeSch40x8.0-cpfV2-14Apr2015 Static CPF Hsg 1-Factor of Safety-Factor of Safety1. Strain.

# Conclusion