|  | Simulation of FFS PipeSch40x8.0-cpfV2 Date: Thursday, April 16, 2015 Designer: FEA Run By Farley F. Shane Study name: 16Apr2015 Actual CPF Hsg Static Study 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh Information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 14. |
| --- | --- |
| # Description 16Apr2015 ACTUAL CPF HSG Static FEA |  |

# Assumptions

# Model Information Model name: FFS PipeSch40x8.0-cpfV2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Mirror1; Solid Body; Mass:16.655 lb Volume:170.744 in^3 Density:0.0975437 lb/in^3 Weight:16.6437 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS PipeSch40x8.0-cpfV2.SLDPRT Apr 16 12:01:30 2015.

# Study Properties Study name; 16Apr2015 Actual CPF Hsg Static Study 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; On. Inertial Relief:; Off. Incompatible bonding options; More accurate (slower). Large displacement; Off. Compute free body forces; Off. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Mass density:; 0.0975437 lb/in^3. Shear modulus:; 3.77098e+006 psi. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Mirror1)(FFS PipeSch40x8.0-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. On Cylindrical Faces-1; Entities:; 2 face(s). Type:; On Cylindrical Faces. Translation:; -0.002, ---, ---. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); 8.41896; 0.290086; 2.68411; 8.84123. Reaction Moment(lbf.in); 0; 0; 0; 0.. On Cylindrical Faces-2; Entities:; 1 face(s). Type:; On Cylindrical Faces. Translation:; ---, ---, 0. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); -22.5589; -5.29299; -0.530004; 23.1776. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 2 face(s). Type:; Apply normal force. Value:; 23289 lbf.. Force-2; Entities:; 2 face(s). Type:; Apply normal force. Value:; 20531 lbf.. Pressure-1; Entities:; 5 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.4 in. Minimum element size; 0.08 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 146189. Total Elements; 75283. Maximum Aspect Ratio; 487.75. % of elements with Aspect Ratio < 3; 70.4. % of elements with Aspect Ratio > 10; 16.5. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:26. Computer name:; ENIGMA.

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -14.14; -5.00287; 2.15506; 15.153. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2719.97 psi Node: 106352; 38775.4 psi Node: 80908. FFS PipeSch40x8.0-cpfV2-16Apr2015 Actual CPF Hsg Static Study 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 4.24618 in Node: 50878; 4.59503 in Node: 7261. FFS PipeSch40x8.0-cpfV2-16Apr2015 Actual CPF Hsg Static Study 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 9.8651e-005 Element: 66281; 0.00230976 Element: 67045. FFS PipeSch40x8.0-cpfV2-16Apr2015 Actual CPF Hsg Static Study 1-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 1.02863 Node: 80908; 3.75 Node: 1002. FFS PipeSch40x8.0-cpfV2-16Apr2015 Actual CPF Hsg Static Study 1-Factor of Safety-Factor of Safety1. Strain.

# Conclusion