|  | Simulation of FFS housingBase-cpfV2 Date: Thursday, April 09, 2015 Designer: FEA run by Farley F. Shane Study name: Base Plate Static Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh Information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 15. |
| --- | --- |
| # Description Static Analysis of Lower Base Plate 09April2015 |  |

# Assumptions

# Model Information Model name: FFS housingBase-cpfV2 Current Configuration: withBellows. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:3.50806 lb Volume:35.964 in^3 Density:0.0975437 lb/in^3 Weight:3.50568 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS housingBase-cpfV2.SLDPRT Apr 08 16:03:12 2015.

# Study Properties Study name; Base Plate Static Study. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; On. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; Off. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Mass density:; 0.0975437 lb/in^3. Shear modulus:; 3.77098e+006 psi. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Split Line1)(FFS housingBase-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Reference Geometry-1; Entities:; 1 edge(s). Reference:; Face< 1 >. Type:; Use reference geometry. Translation:; ---, ---, 0. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); 0; -39854.2; 0; 39854.2. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 3 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.3 in. Minimum element size; 0.06 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 27065. Total Elements; 16551. Maximum Aspect Ratio; 9.4919. % of elements with Aspect Ratio < 3; 98.6. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:03. Computer name:; ENIGMA. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 1 face(s). Units:; in. Size:; 0.3. Ratio:; 1.5..

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; 0.043959; -39854.2; 0.243731; 39854.2. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 649.77 psi Node: 20789; 197650 psi Node: 718. FFS housingBase-cpfV2-Base Plate Static Study-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.0222772 in Node: 475; 0.260422 in Node: 4469. FFS housingBase-cpfV2-Base Plate Static Study-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 6.45085e-005 Element: 7890; 0.0128173 Element: 4430. FFS housingBase-cpfV2-Base Plate Static Study-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 0.201798 Node: 718; 1.25 Node: 117. FFS housingBase-cpfV2-Base Plate Static Study-Factor of Safety-Factor of Safety1. View 1 of FOS. Image Comments:. Locations where FOS is not met.. FOS View 2. Image Comments:. Second view where FOS is not met..

# Conclusion