|  | Simulation of FFS topFlange-cpfV2 Date: Friday, April 10, 2015 Designer: FEA done by Farley F. Shane Study name: 30Mar2015 Top Flange Cap with Holes Static Study 2 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh Information 7 Sensor Details 9 Resultant Forces 9 Beams 10 Study Results 11 Conclusion 20. |
| --- | --- |
| # Description CPF Top Flange 10Apr2015 |  |

# Assumptions

# Model Information Model name: FFS topFlange-cpfV2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line6; Solid Body; Mass:3.53825 lb Volume:36.2735 in^3 Density:0.0975437 lb/in^3 Weight:3.53585 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS topFlange-cpfV2.SLDPRT Apr 08 16:03:13 2015.

# Study Properties Study name; 30Mar2015 Top Flange Cap with Holes Static Study 2. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; On. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Mass density:; 0.0975437 lb/in^3. Shear modulus:; 3.77098e+006 psi. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(9/32 (Split Line6)(FFS topFlange-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 6 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); -0.00871299; 35067.5; 0.00775937; 35067.5. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 13 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 16 Points. Maximum element size; 0.1 in. Minimum element size; 0.02 in. Mesh Quality; Draft Quality Mesh. ## Mesh Information - Details Total Nodes; 159100. Total Elements; 778086. Maximum Aspect Ratio; 213.71. % of elements with Aspect Ratio < 3; 96. % of elements with Aspect Ratio > 10; 0.536. Time to complete mesh(hh;mm;ss):; 00:01:23. Computer name:; ENIGMA. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 11 face(s). Units:; in. Size:; 0.0827772. Ratio:; 1.5.. Control-7; Entities:; 3 face(s). Units:; in. Size:; 0.05. Ratio:; 1.5.. Control-8; Entities:; 67 face(s). Units:; in. Size:; 0.09. Ratio:; 1.5..

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -0.00871299; 35067.5; 0.00775937; 35067.5. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 200.373 psi Node: 29596; 246126 psi Node: 15736. FFS topFlange-cpfV2-30Mar2015 Top Flange Cap with Holes Static Study 2-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 in Node: 15579; 0.0488008 in Node: 151836. FFS topFlange-cpfV2-30Mar2015 Top Flange Cap with Holes Static Study 2-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.72096e-005 Element: 38157; 0.027589 Element: 286256. FFS topFlange-cpfV2-30Mar2015 Top Flange Cap with Holes Static Study 2-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 0.162052 Node: 15736; 1.5 Node: 1. FFS topFlange-cpfV2-30Mar2015 Top Flange Cap with Holes Static Study 2-Factor of Safety-Factor of Safety1. Top VIew Stress 1. Image Comments:. Orange and Red are High Stress Areas. Bottom View Stress 2. Image Comments:. Orange and Red are High Stress Areas. FOS on Bottom of Top Flange. FOS of 1.5 for Top of Top Flange. Image-5. Image-6.

# Conclusion