|  | Simulation of FFS topFlange-cpfV2 Date: Monday, April 13, 2015 Designer: FEA run by Farley F. Shane Study name: Gene Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh Information 7 Sensor Details 9 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 17. |
| --- | --- |
| # Description CPF Top Flange (Gene) |  |

# Assumptions

# Model Information Model name: FFS topFlange-cpfV2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude7; Solid Body; Mass:3.54664 lb Volume:36.3595 in^3 Density:0.0975437 lb/in^3 Weight:3.54424 lbf; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\FFS topFlange-cpfV2.SLDPRT Apr 13 10:21:26 2015.

# Study Properties Study name; Gene. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39885.4 psi. Tensile strength:; 44961.7 psi. Elastic modulus:; 1.00076e+007 psi. Poisson's ratio:; 0.33. Mass density:; 0.0975437 lb/in^3. Shear modulus:; 3.77098e+006 psi. Thermal expansion coefficient:; 1.33333e-005 /Fahrenheit.; SolidBody 1(Boss-Extrude7)(FFS topFlange-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Reference Geometry-1; Entities:; 1 edge(s). Reference:; Face< 1 >. Type:; Use reference geometry. Translation:; ---, ---, 0. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); 0; 36898.7; 0; 36898.7. Reaction Moment(lbf.in); 0; 0; 0; 0.. On Cylindrical Faces-1; Entities:; 1 face(s). Type:; On Cylindrical Faces. Translation:; 0, ---, ---. Units:; in.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); 22.7486; -0.795485; -8.46881; 24.2869. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-3; Entities:; 11 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Curvature based mesh. Jacobian points; 4 Points. Maximum element size; 0.2 in. Minimum element size; 0.04 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 335925. Total Elements; 213417. Maximum Aspect Ratio; 107.75. % of elements with Aspect Ratio < 3; 88.9. % of elements with Aspect Ratio > 10; 2.56. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:32. Computer name:; ENIGMA. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 11 face(s). Units:; in. Size:; 0.1. Ratio:; 1.5..

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; 24.5173; 36898; -7.73346; 36898. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 11.4615 psi Node: 75892; 185498 psi Node: 82000. FFS topFlange-cpfV2-Gene-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.00193302 in Node: 108; 0.0344965 in Node: 180572. FFS topFlange-cpfV2-Gene-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.17031e-005 Element: 109509; 0.0129471 Element: 22984. FFS topFlange-cpfV2-Gene-Strain-Strain1. Name; Type. Displacement1{1}; Deformed Shape. FFS topFlange-cpfV2-Gene-Displacement-Displacement1{1}. Name; Type; Min; Max. Factor of Safety1; Automatic; 0.215018 Node: 82000; 2 Node: 1. FFS topFlange-cpfV2-Gene-Factor of Safety-Factor of Safety1. Top Von Mises. Image Comments:. Local Issues. Bottom Von Mises. Image Comments:. Issues include threaded holes and O-Ring Groove. Bottom Max FOS=2. Image Comments:. Some threaded holes and O-Ring groove are only about 0.6 FOS.

# Conclusion