|  | Simulation of housingBase-cpfV2 Date: Monday, June 15, 2015 Designer: FEA Run by Farley F. Shane Study name: 11June2015 housingBase-cpfV2 Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 13. |
| --- | --- |
| # Description Static FEA for housing Base-cpfV2 Static 1 11June2015 |  |

# Assumptions

# Model Information Model name: housingBase-cpfV2 Current Configuration: withBellows. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude1; Solid Body; Mass:1.57435 kg Volume:0.000583093 m^3 Density:2700 kg/m^3 Weight:15.4286 N; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\housingBase-cpfV2.SLDPRT Jun 11 13:50:30 2015.

# Study Properties Study name; 11June2015 housingBase-cpfV2 Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; On. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75e+008 N/m^2. Tensile strength:; 3.1e+008 N/m^2. Elastic modulus:; 6.9e+010 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2700 kg/m^3. Shear modulus:; 2.6e+010 N/m^2. Thermal expansion coefficient:; 2.4e-005 /Kelvin.; SolidBody 1(Boss-Extrude1)(housingBase-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 edge(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.598572; -152126; 8.61084; 152126. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 14 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.2 in. Tolerance; 0.01 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 56482. Total Elements; 36209. Maximum Aspect Ratio; 13.9. % of elements with Aspect Ratio < 3; 95.5. % of elements with Aspect Ratio > 10; 0.102. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:07. Computer name:; ENIGMA.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.598572; -152126; 8.61084; 152126. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 59.4122 psi Node: 39423; 44530.4 psi Node: 7. housingBase-cpfV2-11June2015 housingBase-cpfV2 Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 mm Node: 253; 0.632832 mm Node: 26587. housingBase-cpfV2-11June2015 housingBase-cpfV2 Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.56989e-005 Element: 13655; 0.00335553 Element: 8597. housingBase-cpfV2-11June2015 housingBase-cpfV2 Static 1-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 0.895688 Node: 7; 5 Node: 23. housingBase-cpfV2-11June2015 housingBase-cpfV2 Static 1-Factor of Safety-Factor of Safety1.

# Conclusion