|  | Simulation of housingBase-cpfV2 Date: Tuesday, June 16, 2015 Designer: FEA run by Farley F. Shane Study name: 16June2015 HousingBase-cpfV2 added o ring Static 2 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 15. |
| --- | --- |
| # Description 16June2015 Static FEA for housingBase-cpfV2 |  |

# Assumptions

# Model Information Model name: housingBase-cpfV2 Current Configuration: withBellows. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Groove for AS 568-215, O-Ring1; Solid Body; Mass:1.56852 kg Volume:0.000580932 m^3 Density:2700 kg/m^3 Weight:15.3714 N; X:\CPF\Mechanical Engr\Farley\CPF2015Feb03\housingBase-cpfV2.SLDPRT Jun 16 08:53:51 2015.

# Study Properties Study name; 16June2015 HousingBase-cpfV2 added o ring Static 2. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; On. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (X:\CPF\Mechanical Engr\Farley\CPF2015Feb03).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75e+008 N/m^2. Tensile strength:; 3.1e+008 N/m^2. Elastic modulus:; 6.9e+010 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2700 kg/m^3. Shear modulus:; 2.6e+010 N/m^2. Thermal expansion coefficient:; 2.4e-005 /Kelvin.; SolidBody 1(Groove for AS 568-215, O-Ring1)(housingBase-cpfV2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 edge(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.732605; -150766; -1.73383; 150766. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 18 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.125 in. Tolerance; 0.00625 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 190202. Total Elements; 128386. Maximum Aspect Ratio; 9.3781. % of elements with Aspect Ratio < 3; 98. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:18. Computer name:; ENIGMA.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.732605; -150766; -1.73383; 150766. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 35.5123 psi Node: 241; 45860.5 psi Node: 184780. housingBase-cpfV2-16June2015 HousingBase-cpfV2 added o ring Static 2-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 in Node: 548; 0.0250169 in Node: 51459. housingBase-cpfV2-16June2015 HousingBase-cpfV2 added o ring Static 2-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 6.92039e-006 Element: 30681; 0.00410179 Element: 15116. housingBase-cpfV2-16June2015 HousingBase-cpfV2 added o ring Static 2-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 0.869711 Node: 184780; 5 Node: 14. housingBase-cpfV2-16June2015 HousingBase-cpfV2 added o ring Static 2-Factor of Safety-Factor of Safety1. FOS. Image Comments:. Interior of Base. FOS 2. Image Comments:. External Face.

# Conclusion