- Goal 1: Make sure we can deploy he MR safely and can run the standard MR processing scripts

  - Oct 8: Deployment 1

    - Depth 105 meters

    - Anchor on the bottom

    - 5 profiles

    - Microrider mounted with 2 probes

  - Science Analysis

    - Verify the noise characteristics of the SBE41 pressure sensor

    - More

- Goal 2:  Determine the safe mission duration (2 deployments)

  - Week of Oct 11-15 and one more in the same week of the next week:

  - Same as above with max profiles

- Goal 3: Verify parking capability (1 deployment)

  - Park at 90 meters in 105 meters of water

  - 5 profiles

- Goal 4: Verify parking capability with max mission duration ( 1-2 deployments)

  - Park at 90 meters in 105 meters of water

  - Max profiles

- Goal 5: Verify max depth capability

  - In parallel with 1-4 Gene will review calcs and potentially pressure test a spare housing

  - No MR.  1 profile parking at 150 meters in 175 – 250 meters of water

  - Check internal humidity and pressure and whatever else makes sense.  Maybe recover an inspect

  - Repeat 5.1-5.3 at 200 and 250 meter parking depths.

- 1 day of  profiles at M1

- Max number of profiles at M1 parking at 250 meters 2-3 deployments