CPF-P2 Deployment check list				Date:

User:

Mission Statement:

- Software Checks

_____ Clear breakpoints					_____	Comment out compiler directives

- Mission Configuration

_____________ Mission TimeOut (HH:MM)		_____________ Park Sample Period (HH:MM)

_____________ Max Profiles				_____________ Max Pressure (m)

_____________ Park Depth (m)					_____________ Ascend Glide Depth*

_____________ Platform Velocity Descend (m/s)		_____________ Platform Velocity Ascend (m/s)

_____________ Park/Anchor Time (HH:MM, Descend Table)

_____________ Estimated Surfacing Time (HH:MM)

_____________ Pre-Mission Delay TimeOut*			_____________ Surface Ops TO*

_____________ Descend TimeOut* 			_____________ Park TO*

_____________ Anchor TimeOut*				_____________ Ascend TO*

________________ Homer ID

- Pre-Deployment Mechanical Checks

_____________ Can Humidity (% RH)			_____________ Can Pressure (mbar)

_____________ Battery Bus Voltage (V)		_____________ Battery Bus Current (mA)

_____________ Bellows Position Limits*		_____________ Number of Ballast Bars*

_____________ SurfaceOps Bellows Position	_____________ RecoveryMode Bellows Position

_____Receive SBD message				_____Receive GPS Tracker Message (if mounted)

_____Double check lifting bridle			_____Remove Optode  cover

_____Remove CTD caps					_____Remove FLBB cover

_____Remove ISUS cover				_____Connector dummy plugs installed

Gene to count__Number of ballast bars	           _________________Battery bus voltage

___________________Humidity/Temp.		_________________Int. pressure/Temp.

_____Remove Telonics Magnet (Freq 160.15000MHz)

_____Homer Beacon (ID 13 (07-1))

- Deployment

__________________ Water Depth (m) 		__________________ Over the side Time

__________________ Mission Start Time (On)		__________________ Under Surface Time (UT)

__________________ Profile Start Time ($GO)	__________________ Descend Time

__________________ Deployment Lat (UT)			__________________ Deployment Lon (UT)

FORM ON BACK FOR SUBSEQUENT DIVES FROM STARTING PROFILE:

| Profile # | Under Time | Lat/Lon Under | Surfacing Time | Lat/Lon Surface | Surface Drift | Drift at Depth | GARMIN |
| --- | --- | --- | --- | --- | --- | --- | --- |

*

Done by Gene or Eric

Look out for “Creating Mission” parameters and Descend/Ascend Table in TeraTerm Console output.

*To be printed out

Note: Sets the clock for MissionTimeOut.