Coastal profiling float annual report

The coastal ocean plays a disproportionate role in the global carbon cycle, accounting for ~50 % of annual new phytoplankton production despite only covering 5-10 % of total ocean area. These high levels of biological activity significantly influence local patterns of air-sea CO2 exchange, and the associated biomass supports a majority of global fishery production. However, coastal carbon cycling remains poorly quantified due to the challenges associated with observing such dynamic regions. For example, short-term events such as hurricanes, storm water discharge, upwelling, and phytoplankton blooms impact local carbon cycle processes. Traditional ship-based strategies are impractical for observing these stochastic events, which may contribute significantly to regional budgets. As a result, a recent Coastal Carbon Synthesis report highlighted the need to further develop our observational capacity through the development of new autonomous platforms specifically designed to work in the continental margins.

MBARI’s Coastal Profiling Float (CPF) project has developed a new autonomous platform optimized to make biogeochemical measurements in the continental margins (the coastal zones) of the world’s oceans. Profiling floats are arguably the only platform that can (and have) scaled up to support global and regional scale observing projects that extend beyond the surface on an economically sustainable basis. The international Argo project is aimed at quantifying the heat content of the world’s ocean and has maintained a global array of more than 3000 profiling floats for more than a decade. The SOCCOM project is deploying 200 floats in the Southern Ocean to determine its influence on the global carbon cycle and climate. Profiling floats move vertically through the water column by changing their buoyancy. The buoyancy “engine” in the CPF has roughly 10 times the capacity of the floats used on Argo and SOCCOM allowing the CPF to 1) profile through the strongest density gradients seen in the ocean, e.g., the Columbia River mouth and in the Arctic, and 2) anchor on the sea floor to reduce horizontal drift outside the desired measurement domain. In addition, the CPF supports the desired suite of biogeochemical instruments (salinity, temperature, pH, oxygen, nitrate, fluorescence, backscatter and solar irradiance) with the capability to support additional instruments. The CPF is proving to be a versatile platform and the MBARI Biological Oceanography Group is using 2 CPFs with significantly larger payloads, including a sediment trap and acoustic telemetry, to study particle export dynamics in the Monterey Bay. Combining particle and biogeochemical data will provide a more comprehensive understanding of coastal carbon cycle processes.

The CPF project has gone through an extensive development and testing process over the last several years. In 2018, the project will transition from development into science operation. The first science mission is planned for the late summer of 2018, where 3 CPFs will be deployed and operated simultaneously in an array to investigate the source of nutrients supporting elevated primary production in Monterey Bay. This region in the northern Bay has been termed an upwelling incubator (Ryan et al., 2008).  A strong nutrient supply is required to support such systems.  Lateral transport of upwelled water from the Monterey Canyon has been suggested as one significant nutrient source for this system (Ryan et al., 2010).  The floats will act as a small mooring array that can detect movement of nutrient rich water. The study is expected to last 1 month, with each CPF performing multiple profiles per day. A key technical objective of the mission is to incorporate high-resolution model output of regional ocean currents to autonomously control the array geometry and location throughout the experiment. An MBARI Wave Glider equipped with instrumentation for measuring surface air and sea carbon dioxide will follow the floats during this field test. Comparisons between CPF and Wave Glider carbon data will inform future field studies. This experiment will be conducted through a collaborative effort between K. Johnson, A. Fassbender, and Y. Takeshita, and represents the integration of young scientists into MBARI research through strong mentorship from senior scientists and engineers. As a result, in addition to the Biological Oceanography Group led by F. Chavez, there are four groups actively utilizing CPF technology, highlighting the versatility of this new platform. Thus, the CPF not only represents a potential foundation for a global in situ coastal ocean observing system, but it is playing a key role in forming strong institutional collaborations at MBARI to conduct multidisciplinary studies.

The Coastal Profiling Float

The CPF Configured for the Biological Oceanography Group

Examples of Standard CPF Data Products