# CPF Production Checkout Form

Revision 2

Unit SN:

### Installation

#### LHE (SN: 003)

6 Batteries Installed

SB1-10 unconnected

JP1 removed for Sleep enabled

JP2-3 2-2 for logic control

Fuses installed (3 places)

#### Motherboard (SN: 05)

SD Card Inserted (Size: 8GB)

JP1, JP2 Removed (LED Disable)

JP3 MODE wired for USB MODE

USB client harness installed

#### Daughtercards

SN2 in SLOT-1 (SN: P70726)

SN1 in SLOT-2 (SN: P70723)

SN3 in SLOT-3 (SN: P70731)

BTB on mount: Configured for RS232, LED resistors removed,

(SN: 03)

ISB in SLOT-6 (SN: 1)

#### Upper Assembly Hardware

MSC/ISUS/DuraFet (SN’s: P38956)

SBE Board (SN: P118457)

GPS (SN:      )

Iridium Modem

(SN:      )

(IMEI: 1300125010615650)

(SIM:      )

GPS/Iridium subsystem deck test

#### Lower Assembly Hardware

String pot & ADC (SN: 36928)

ELMO & Motor (SN: GSL17520092)

Motorized Valve

Arrow pointing toward pump

Bellows Plumbed

#### Wire Harness Check

WH-Battery Charger

WH-BT Console

WH-CTD Pwr/Comms

WH-FLBB

WH-GPS

WH-Iridium Comms

WH-LHE to MoBo Pwer/ISB

WH-Mot Valve

WH-MSC

WH-other msc

WH-OCR504

WH-On/Off Switches

WH-Optode

WH-Pos Xducer

WH-PTH

WH-Pump Motor

WH-ucComms

Finalize all WH routings

### Pre-oil Fill

Motherboard SW Checkout

Energy Monitor BATT

12V Control

5V Control

#### Sensor Functionality

SLOT1-J5 to SLOT5 BT Console PWR/TELEM

SLOT1-J2 CTD PWR/TELEM

SLOT1-J4 Pump PWR/TELEM

SLOT2-J2-J3 Iridium PWR/TELEM

SLOT2-J4 MSC PWR/TELEM

SLOT2-J5 Optode PWR/TELEM (SN: 2707)

SLOT3-J4 OCR PWR/TELEM (SN:      )

SLOT3-J5 FLBB PWR/TELEM (SN:      )

SLOT6-J1 GPS PWR/TELEM (SN:      )

SLOT6-J2 Bellows Position PWR/TELEM

SLOT3-J6 Mot Valve PWR/DIO (SN:      )

SLOT6-J8 LHE I2C Attached

#### LHE

LHE Battery Polling

LHE Sleep Test

LHE Pump PWR/TELEM

LHE Pump Energy Monitor

Charging Cycle through Endcap

#### Calibrations

String Pot Calibration:

100% = 3mm below upper plate

0% = bellows top against bellows bottom

SLOPE:<slope> INTERCEPT:<intercept>

MSC Calibrations

Environmental Sensors on ISB

#### Closed Unit Deck Test

Iridium Deck Test

GPS Deck Test

Bluetooth Deck Test

### Oil Filled

#### Benchtop

Fill Oil

Pump/Motor Calibration

Pump Bellows Cycling (10x)

Install desiccant packs

Close Unit

### Tank Tests

#### Surface Operations

Bluetooth

Temperature & Humidity Good (<25%RH)

GPS [ATTEMPT]

IRIDIUM SBD [ATTEMPT]

IRIDIUM SBD [ATTEMPT]

#### Buoyancy Operations

Ballast Tests

SurfaceOps Bellows Position

Descent/Ascent

Velocity Control

Depth Holding

Profile Control

Sampling Profile

### Configuration File Options

TBD