CPF Inertial Current Profiler Requirements

Top level Goal

Determine the 2 dimensional horizontal velocity profile (speed and direction vs depth) of the CPF as it ascends from its park or anchor state to the surface.  The goal is to identify a depth with a current roughly toward the desired horizontal position (latitude and longitude) for the next CPF profile.

Specific requirements

- Using a suitable inertial measurement unit, the Inertial Current Profiler (ICP) shall measure accelerations and other necessary data (gyro, compass, etc.) and perform the calculations necessary to provide a table of current direction and speed vs depth.

- Units shall be as follows.

  - velocity: meters per second

  - depth: dBar of pressure

  - direction: compass degrees relative to true north.

- The final current profile shall be averaged into bins with a user configurable resolution from 1 dBar to 20 dBar.

- All ICP data from each profile shall be stored on board the ICP. SD, microSD or USB flash drives are all acceptable mass storage devices.

- The velocity profile does not need to be calculated in real time, i.e., it can be calculated at the end of the CPF ascend state.

- The logger shall be independent of the CPF electronics with the following exceptions.

  - Power for the logger can be drawn from the CPF battery bus (24 to 36 VDC).

- The ICP shall monitor the CPF Bluetooth signal to determine the “start of ascend” and “end of ascend”.

- The ICP shall monitor the CPF Bluetooth signal to get current CPF depth.

- The ICP shall provide the final current profile to the CPF via Bluetooth with a format TBD.

- As estimate of the ICP’s performance shall be provided based on analysis and test data.

- As a goal, the following functionality may be provided

  - Running the ICP periodically or continuously during the anchor state to determine the accelerometer drift is desirable

  - Running the ICP during surface ops and calculating a speed and direction for comparison with the GPS fixes is desirable.