GDF Assembly Instructions

Initial Draft	2021 Aug 03	Gene Massion

- Documents

All drawing are in \\atlas.shore.mbari.org\projectlibrary\ChemSensors\GeneM\GDF pH Sensor\Production\AssemblyDrawings

  - Bill of Materials

  - Exploded GDF drawing

  - Van London Pellet AID

  - CCA Assembly drawing

  - …

- Tools

  - ¼” drive torque wrench

  - VL pellet installation tool

  - GDF inspection plate

  - GDF housing broaching fixture

  - x/x” hex socket

  - y/y” hex socket

  - …

- Assembly materials

  - O-ring lube: CRC food grade silicone oil p/n xxx, Parker Super Lube p/n yyy, ???

  - Loctite superglue: P/N xxx

- Assembly steps

  - Machine the VL pellet and bag individually

  - Broach the ISFET recess corners in the housing

    - Photograph all 4 corners and log

  - Mark housing with serial number

  - Go/No-go the fuzz buttons by length

  - Kit the parts

    - CCA parts in one bag

    - VL pellet in one bag

    - Anything with an O-ring surface should be individually bagged by the vendor

    - Other stuff?

  - Inspect the parts

    - Log housing dimensions in production log

    - O-ring surfaces

    - SAE port flatness

  - Build up the CCA

    - Inspect the back side for flatness

  - Install the VL pellet solder pin

  - Install the VL pellet fuzz button and O-ring

    - Inspect the O-ring

    - Lube with xxxx

  - Install the VL pellet with the installation tools

    - Photograph and log

    - Torque the 2X pellet cover screws per drawing

  - Install the ISFET

    - Make sure it sits flat

    - Check ISFET chip height relative to housing face

  - Install the gasket on the front cover shoulder

    - Ensure the gasket doesn’t ooze under the shoulder

  - Install the front cover and gasket

    - Clean, inspect, lube and install the front cover O-ring

    - Ensure the gasket doesn’t ooze under the shoulder

    - Photograph front cover opening and log

    - Torque the 4X cover screws per drawing

    - Inspect cover plate is flush with housing

  - Install the ISFET fuzz buttons

    - Photograph and log how proud they sit above the housing face

  - Install the CCA

    - Make sure the fuzz buttons get pushed into the hole, not smeared under the CCA.

    - Photograph and log

  - Temporarily install port adapter to float head

    - Mark orientation for GDF installation

    - Remove port adapter from float head

  - Install the Pave penetrator in the port adapter

    - Install coax on low pressure side of penetrator

    - Cut …color… wire on both sides of penetrator

    - Twist wires as shown on drawing

    - Check wire length on both sides of penetrator per the drawing

    - Inspect the penetrator and O-ring

    - Lube O-ring and install penetrator. Ensure high pressure side of penetrator is facing GDF

  - Assemble GDF and port adapter

    - Twist GDF X turns to “slinky” wires on the high pressure side of the penetrator

  - Solder penetrator wires to CCA

  - Solder VL pellet wire to CCA

    - Photograph and log CCA installation

  - Install the back cover plate

    - Clean, inspect, lube and install the back cover O-ring

    - Make sure the back cover plate is flush against the housing

  - Next steps …