CPF Deployment check list

Gene Massion

Version 1.0 	2014 August 31

- Software

  - Make sure stuff used in tank testing is changed for deployment

    - Program.cs

      - Set initial state at top of State Machine while loop

      - uBlox.Request fix number of tries and time delay in SurfaceOps

    - configFile.cs

      - Set appropriate preMissionDelayTimeout

      - Set appropriate SurfaceOpsTimeout

      - Uncomment SOSetBellowsPosition to correct value

    - Clear all breakpoints

    - Comment out debug statements

  - Mission Config

    - Make sure last ascend has 0.0 depth

    - usbCable = false

    - Set SBE41 pump time

    - Set SBE41 cpCutoff pressure

  - Set SORecoveryMode as appropriate

- Balllast so mark on antenna stem is at waterline with bellows at SOSetBellowsPosition

- Remove eye bolts as necessary

- Remove Optode rubber cover

- Remove CTD caps

- Check tension screw cover is tight

- Check Swagelok fitting is tight

- Check tapered ring/bellows cover are screwed together tightly

- Check lifting plate is secured properly

- Check Optode and CTD settings are correct