- Elmo set baud rate

  - PP[2] = 4   (57,600 baud)

  - PP[2] = 3 (38,400)

  - SV (save paramaters)

  - Power down elmo

  - Should be stuck in new baud rate at power up.

- Test tank mission

  - Turn the power on

  - Button up the pressure housing

  - Program initializes into idle state with desired depth = 7 psi (~5 meters)

  - Put the float in the tank

  - (optional) Send a new “set target depth” command

  - Send a “Go” command

  - Float goes to desired depth with +/- 0.7 psi (0.5 meter) dead band

  - Maintain depth (+/- 0.7 psi) for 2 minutes

  - Surface (what is surface?)

    - Wait for “stop” command

  - Back into idle state

  - Wait for new command, “set target depth”, “go”

- PID Benchtop simulation

  - Setup calibrator (or whatever) to simulate 4-20 mA pressure loop.  4 mA = 0 psi, 20 mA = 15 psi.

  - Start PID loop running with target depth = 7 psi and calibrator = 0 mA.

  - “Log” y2, y1, v, u, I.  y1 is the filtered version of the measured pressure signal

  - Increase “pressure” to 7 psi over 50 seconds.

  - The speed command should decrease to near zero as the “pressure” approaches 7 psi.  The I term will jump around

  - Hold for 1 minute

  - Stop PID loop

  - Do it again adding some noise (maybe by toggling one wheel of the calibrator).