Customized Battery - Technical Requirement Form  _________

*REQUIRED Parameters (need to be filled out in order to ensure proper processing of your inquiry)  Date:

| Date |  | October 14, 2013 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Customer |  | Gene Massion |  |  |  |  |
| Project Name |  | CPF Li-Ion Battery Pack |  |  |  |  |
| Application |  | Oceanographic instrumentation |  |  |  |  |
| Estimate Annual Volume |  | 10 |  |  |  |  |
| Target Unit Price |  | $200 |  |  |  |  |
| *Operating/Working Voltage Range Of The Device To Be Powered |  | *Minimum Voltage | Nominal Voltage |  | *Maximum Voltage |  |
|  |  | 24 | 29.6 |  | 38 (on charger) |  |
| *Average Discharge Current |  | *Maximum/Burst Discharge Current |  | 3 A total from 6 packs in parallel |  |  |
| *How long does each burst last (seconds) |  | 600 |  |  |  |  |
| *Average Charge Current | 3A | Max Charge Current / Time |  |  |  |  |
| *Minimum Capacity of Battery (mAh/Ah) |  | 5.2 Amp - hours |  |  |  |  |
| *Operating Temperature Range (Please Specify Degrees Fahrenheit or Celsius) |  | *Minimum: | Average: |  |  | *Maximum: |
|  |  | 4 degC | 25 degC |  |  | 35 degC |
| Desired Connector (Manufacturer / Part Number) |  | TE 794617-2 (Digikey A30293-ND) |  |  |  |  |
| Wire Specifications (length, color, gauge, material…) |  | 15 cm |  |  |  |  |
| Maximum Dimensions Of Battery (Specify in either mm or inches.) |  | 265mm x 40mm x 36mm (similar to All-Battery 31801) |  |  |  |  |
| Casing Requirement (PVC, hard metal /plastic casing) |  | Similar to All-Battery 31801 |  |  |  |  |
| Preferred Cell Vendor |  |  |  |  |  |  |
| Minimum Number of Charge/Discharge Cycles |  | 100 |  |  |  |  |
| Required Safety Certification (CE / UN / UL / FCC) |  |  |  |  |  |  |
| Other Requirements: - Similar to All-Battery 31801 except 8S-2P with appropriate protection PCB and poly switch - A total of 6 of these packs will be diode or’d in parallel. All 6 packs will be charged in parallel on the battery side of the or’ing diode. |  |  |  |  |  |  |