### Closed-loop buoyancy control for a Coastal Profiling Float

### Laughlin Barker, Santa Clara University

##### Mentor:  Gene Massion

## Summer 2014

# Keywords:  profiling float, buoyancy engine, control, PID, root locus, linearization

# ABSTRACT

Abstract text (Body text 2, Times New Roman, 12 pt; 1.5 line spacing)

# NOMENCLATURE

| FB | Buoyant Force [N] | CD | Drag Coefficient |
| --- | --- | --- | --- |
| FG | Gravitational Force [N] | A | Float Frontal Area [m2] |
| FD | Drag Force [N] | g | Gravitational Acceleration, -9.81 m/s2 |
| V | Instantaneous Float Volume [m3] | clin | Velocity linearization parameter |
| V0 | Static Float Volume [m3] | a, b | Transfer Function Coefficients |
|  | Buoyancy Engine Flow Rate [m3/s] | u | Control Signal |
| m | Float Mass [kg] | VSP | Velocity Set Point [m/s] |
| ż | Velocity [m/s] | IB, OB | Velocity Control Bands [m] |
| ρsw | Sea Water Density [kg/m3] |  |  |

# INTRODUCTION

Coastal waters (herein defined by the continental shelf), are highly productive environments. While occupying only 7-10% of total ocean area, it is estimated they are responsible for 40-50% of net primary productivity in the ocean (Bauer et al. 2013, Ducklow & McCallister 2004). In a study of oceanic and terrestrial systems, Geider et al (2001) estimate that globally, coastal waters are responsible for approximately 10% of the Earth’s net primary productivity.

While undoubtedly important, a lack of appropriate temporally and spatially scaled data has left our understanding of these systems with a high degree of uncertainty (Bauer et al 2013). High cost has prevented historical biogeochemical studies (research cruises) from being scaled to capture the heterogeneous nature of these waters. In the last ten years however, low-power biogeochemical sensors (pH, Oxygen, Nitrate, pCO2 and others), have been developed and are being proven to be suitable for long-term observation systems (Johnson et al 2009). With the advent of these sensors, the primary technical challenge of implementing a coastal observing network lies in cost effective long-term deployment.

Perhaps the single tenable option for a scalable biogeochemical observing network is through the use of profiling floats. The Argo Array, launched in 1999 currently sports over 3,600 Apex Profiling Floats which profile the top 2,000 m of the world’s oceans on average every 10 days.  The profilers have a battery life of 4-5 years, and telemeter collected data via satellite during surface intervals. The life-cycle cost, including Iridium/Argo satellite fees, of a single Apex float is approximately $30,000 (Argo.ucsd.edu).

While the underlying technology of profiling floats has been successfully scaled to a global deployment, several challenges of working in shallow coastal waters preclude the use of commercially available profilers in coastal environments. Figures 1 (a) and (b) show the trajectory and maximum depth of an Apex float operating in the Pacific Ocean. Excessive drift and coarse depth control make operating these floats in small and shallow (< 500 m) waters difficult.

| ~860 km (a) | (b) |
| --- | --- |
| Figure 1: Apex (Argo) profiling floats often exhibit large drift and coarse depth control, making them unsuitable for operation in coastal waters (which are small and shallow by nature). The 1,016 day trajectory of a single float can be followed with the white line in (a). The start pressure of each profile is shown in (b) and shows the coarse nature of the float’s depth control. |  |

To address these challenges, in 2012 the MBARI Chemical Sensors Laboratory began development of the Coastal Profiling Float (CPF). To minimize drift, the CPF is designed to “anchor” itself by becoming negatively buoyant and resting on the benthos between profiles. Closed-loop control buoyancy control will ensure accurate platform velocity when profiling, and fine depth control should it park itself in the midwater.

The process, details, and test results of designing a closed-loop (feedback) velocity and depth controller for the CPF are presented here. (improve transition).

###### MATERIALS AND METHODS

CONTROL STRATEGY

Because control of both velocity and position of the CPF are required, it was decided to design and implement a cascaded velocity and position controller. By designing a velocity profile with respect to displacement from a desired position, cascaded control strategies allow one to control a plant’s velocity and position. Such controllers are commonly used in industrial CNC  machine tools (Karlsson 2009 & Mandra 2014). In Figure 2 two the general structure of the closed-loop control system is shown. The target depth, velocity set point, controller, control signal, plant model, and depth are represented by, r, Vsp, Gc(s),u, Gp(s), and y respectively.

Figure 2: To control the CPF, a cascaded velocity-position controller is implemented. Position of the float relative to its target depth adjust the velocity set point. A controller acts on the velocity error, and adjusts accordingly.

PLANT MODEL

To design the velocity controller, it is first necessary to model the dynamics of the float. Figure 3 shows the free body diagram of a profiling float during ascension.

Figure 3: To model system dynamics, a free body diagram is drawn with acting forces buoyant, gravitational, and drag forces, FB, FG, and FD respectively. The body has some mass, m, volume, V, and velocity, v.

The float has some mass, m and volume, V, velocity, v, with the seawater having a density, ρsw. The buoyant, drag, and gravitational forces are given by:

where V0 and  are the float’s fixed volume, and buoyancy engine’s volumetric flow rate; the time integral of  gives the buoyancy engine’s contribution to the total system volume. The velocity component of the drag term is given by   and not  so the sign of  is respected. The buoyancy term can be simplified by assuming the constant buoyant force is equal to the gravitational force of the float, resulting in a simplified equation of motion,

This equation of motion is used as the basis for a full dynamic simulation in MATLAB Simulink. As implemented, the model includes features such as: realistic sea water density profiles computed from MBARI ROV Ventana CTD data, added mass factor, pump (in)efficiency, ideal-gas law compressibility to model air trapped in the buoyancy engine, and the linear drag experienced by an object moving slowly (Re < 100) through a stratified fluid as described by D’Assaro 2003.

For development of the velocity controller however, the non-linear differential equation of motion, (the nonlinearity due to quadratic drag) is linearized. Drag is linearized such that the linear approximation, , yields an equal drag force at zero velocity, and some nominal velocity, vnom,

Figure 4: To perform a Laplace transform on the equation of motion, the non-linear drag term is linearized at the zero velocity point, and some nominal platform velocity, vnom, shown here as 0.1 m/s. The quadratic drag was computed with ρsw = 1027 kg/m3, CD = 1.5, and A = 0.1018 m2.

Grouping constant terms,

the simplified equation of motion is transformed into the Laplace domain, and open-loop transfer function, Gp(s), found,

Figure 5: The open loop transfer function given by Equation XX has two poles, located at s = 0 and s = -b/m.

TEST TANK PROTOTYPE

A Test Tank Prototype (TTP) of the CPF was built in early 2013 as a proof of concept, and served as the test platform for the development of the controller. Figures 6 (a) shows the prototype, with relevant system components listed in 6 (b).

| (a) | m; 38 kg. Adjustable Buoyancy; 3 L. Power; Qty 6: Tenergy Li-Ion 25.9V 2600mAh. Micro Controller; EMX System on Module (.NET Micro Framework w/C#). Motor; Maxon RE65 w/Quadrature Encoder. Motor Controller; Elmo Solo Whistle. Pump; Oildyne 692. Pressure Transducer; Honeywell 20psi 4-20mA. Pressure Transmitter; Omega DGH D1000 Series 4-20mA A/D. (b) |
| --- | --- |
| Figure 6: The Test Tank Prototype used during testing of the control system is shown in (a). Relevant power, control, and platform parameters are listed in the table shown in (b). |  |

CONTROLLER

A discrete PID controller with second-order low-pass filter had been previously implemented in C# on the CPF’s microcontroller. The controller and filter is based on that found in Astrom and Hagglund 2006. In its continuous form, the controller is modeled by,

with the filtered input signal, yf comes from a second order low-pass filter. The transfer function of this filter is,

where Tf is the time constant of the filter. This filter has a damping coefficient of 0.707, corresponding to ~4.3% overshoot (this is considered a good compromise between fast response time, and minimal overshoot in control systems design). The filter can serve not only to filter out higher frequency electronic noise that occurs on the pressure transducer line, but also sinusoidal pressure variations that arise due to wave action. Figure 7 below shows a spectrogram of waves for different wind speeds. Based on this graph, it was decided that a 3 dB cut off frequency of 0.1 Hz (Tf = 1.59) would minimize influence to the system by most waves, without introducing excessive measurement delay into the system. The settling time, Ts for the 0.1 Hz filter is approximately 9.6 seconds.

Figure 7: Wave frequency varies with amplitude, which is influenced by wind speed. Plotted here is the wave spectrogram for different wind speeds in fully developed seas, according to Moskowitz (1964). A 3 dB low-pass frequency of 0.1Hz was chosen and implemented in the filter.

The overall transfer function of the PID controller and second order filter, Gc(s), is given by

plant model was imported into the MATLAB sisotool in order to facilitate Root Locus analysis of the controller.

As a starting point, existing gain values (K = 70, TD = 10, TI = 50,000) were used in the velocity controller. Modeling done previously seemed to indicate the controller was stable (likely over damped), and achieved a settling time of ~90s – relatively slow in the context of many dynamic mechanical systems, but acceptable for a profiling float in the ocean.

The velocity set point controller is broken into three separate bands: mission velocity, approach velocity, and zero velocity. Mission velocity is the primary velocity at which the float ascends or descends between depth waypoints. Approach velocity is an appropriately small velocity to help “guide” the float to the appropriate depth, and return it to that depth should it leave the park area unexpectedly.

Figure 8: The velocity set point controller for the CPF is discrete instead of continuous. When the float is far away (> OB m) from its depth set point, the mission-defined velocity is the set point. Once the float is inside the yellow band, bounded by IB and OB, the set point velocity is adjusted to the approach velocity (0.01 m/s). Finally, once the float enters the park depth, DSP+/- IB, velocity is set to zero. This approach allows for some settling and provides a dead band to prevent oscillation when in the park zone. Should the float drift out of the park zone however, it will be commanded back with a slow velocity “nudge” to put it back inside.

# RESULTS

SIMULATION

The simulation was run with the added mass factor (25%) and CD= 1.5 found by Sohn, 2013. Simulation results shows the controller should be capable of achieving a zero, or near zero velocity of the float. The Root Locus analysis of the closed loop system, Figure 9, with the above noted constants showed that the dominant system poles stayed on the real axis, indicating there should be no oscillation expected when given a step input.

Figure 9: The SISO tool produced Root Locus and Bode responses for the closed loop velocity controller. Dominant (slow, or close to the imaginary axis) poles have no complex component, so no oscillation is expected when the system is given a step input. Likewise, Gain and Phase Marin of 24 and 93 dB respectively indicate the controller is stable given the physical parameters of the plant.

Response to velocity input signal was generally good, though somewhat difficult to measure. Sample time for the controller and pressure transducer was 1s, and the 4-20 mA to RS-232 converter used with the pressure transducer provided approximately 0.065 m resolution. Pressure samples were put through a differentiator filter, which in turn estimated velocity. Figure 10 shows the command and response of the system to the velocity set point over the course of a dive.

Figure 11: Blue lines show the commanded velocity over the course of a dive profile, and the red plots show the differentiator filter estimation of velocity. Because pressure samples were taken at 1Hz, and the pressure sensors & A/D combination had a resolution of 0.065 m, sequential pressure measurements could only provide coarse velocity measurements. It is worthy to note that when commanded to maintain zero velocity, the platform appears to show slight oscillator behavior about a zero velocity. One possible reason for this is that the drag linearization provides a poor estimation of drag at low velocities.

Figure 12: The depth profile of a dive to 7 and 4 m, with two 300 s holds is shown. The commanded descent and ascent velocity was 0.05 m/s, with IB = 0.2 m and OB = 1 m. Both the model and the test showed oscillatory motion when trying to hold zero velocity. While deviation in position are small, they can be seen with greater resolution in the velocity measurements of the float (Figure 11). The superimposed yellow and green bars depict the approximate velocity set point thresholds.

### DISCUSSION

The cascaded control approach to shows good ability to regulate both position and speed, as can be seen in Figure 12. This should come at little surprise assuming a stable velocity controller that isn’t subject to excessive delays from filtering, or slow rise time, which could result in excessive phase shift, and resultant instability.

### CONCLUSIONS/RECOMMENDATIONS

Text (Normal, Times New Roman, 12 pt; 1.5 line spacing)

# ACKNOWLEDGEMENTS

I would like to acknowledge Gene Massion for his guidance over the course of my internship. His years of experience in the field of ocean engineering and light hearted nature make him one of the most enjoyable, and accessible experts in the field. I also thank Wayne Radochonski, for helping me through C#. To Paul Coenen, Jonny Ferreira and Jim Montgomery for their help with all things mechanical and electrical on the Test Tank Prototype. I also thank Ken Johnson and the rest of the Chemical Sensors Lab for welcoming me into the lab for the summer; you all are doing some truly exciting things! To George Matsumoto and Linda Kuhnz, I wouldn’t have had this opportunity were it not for your vision and dedication to the internship program. Finally to the David and Lucile Packard Foundation.

# References:

http://www.argo.ucsd.edu/FAQ.html