|  | Simulation of bellowsFlange Date: Thursday, March 01, 2012 Designer: Solidworks Study name: Study 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh Information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 13. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: bellowsFlange Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. #10-32 Tapped Hole2; Solid Body; Mass:9.32245 lb Volume:95.572 in^3 Density:0.0975437 lb/in^3 Weight:9.31612 lbf; C:\PDMWorkingDirectory\bellowsFlange.SLDPRT Mar 01 09:08:31 2012.

# Study Properties Study name; Study 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SolidWorks Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SolidWorks document (C:\PDMWorkingDirectory).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75e+008 N/m^2. Tensile strength:; 3.1e+008 N/m^2. Elastic modulus:; 6.9e+010 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2700 kg/m^3. Shear modulus:; 2.6e+010 N/m^2. Thermal expansion coefficient:; 2.4e-005 /Kelvin.; SolidBody 3(#10-32 Tapped Hole2)(bellowsFlange). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); -3.65876; -108160; 0.788173; 108160. Reaction Moment(lbf-in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 11 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi.. Pressure-3; Entities:; 16 face(s). Type:; Normal to selected face. Value:; 750. Units:; psi..

# Connector Definitions No Data

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.3 in. Tolerance; 0.015 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 100586. Total Elements; 57947. Maximum Aspect Ratio; 14.935. % of elements with Aspect Ratio < 3; 86.6. % of elements with Aspect Ratio > 10; 0.337. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:23. Computer name:; MYOJIN. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 16 face(s). Units:; in. Size:; 0.05. Ratio:; 1.5..

# Sensor Details No Data

# Resultant Forces ## Reaction Forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -3.65876; -108160; 0.788173; 108160. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf-in; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 60.5206 psi Node: 6658; 36776.4 psi Node: 86321. bellowsFlange-Study 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 in Node: 1764; 0.0242477 in Node: 2287. bellowsFlange-Study 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.48778e-005 Element: 19704; 0.00345981 Element: 38647. bellowsFlange-Study 1-Strain-Strain1. Name; Type. Displacement1{1}; Deformed Shape. bellowsFlange-Study 1-Displacement-Displacement1{1}.

# Conclusion