Coastal Profiling Float (CPF) Software Statement of Work

June 25, 2012

Gene Massion

- Overview

This document will describe the software tasks we’d like to accomplish in order to start at sea engineering testing of the new CPF currently planned for 2013.  Tasks we’d like to accomplish in 2012 with the approximately 8 weeks of Wayne’s time we’ve been given are identified.  A more detailed description of the software functionality requirements is given in a separate document.

| - Milestones |  |
| --- | --- |
| - Design top level software architecture | 2012 |
| - Including engineering log information |  |
| - Implement enough of the SA to allow buoyancy engine testing | 2012 |
| - Gene’s software in Wayne’s architecture - Use Elmo controller to start, Scott’s controller eventually |  |
| - Tank tests of buoyancy engine (BE) (Gene, not Wayne) | 2012 |
| - Implement CTD driver | 2012 |
| - Implement GPS driver | 2012 |
| - Transfer CTD, GPS and engineering log data over Iridium |  |
| - With analog modems | 2012 |
| - With Iridium modems | 2012 (maybe?) |
| - Implement Accelerometer driver | 2012 (maybe?) |
| - Initial Engineering deployments | 2013 |
| - Implement mission control agent | 2013 |
| - Re-do Gene’s BE code | 2013 |
| - Implement instrument sampling agent | 2013 |
| - Implement remaining instruments drivers (order TBD) | 2013 |

- Instrument list

  - Seabird 41CP CTD http://www.seabird.com/alace.htm

Note that this sensor is somewhat unique in that it has the pressure sensor and will need to be sampled in order to get the pressures needed for depth control.

  - MBARI pH sensor (Ken Johnson)

  - Oxygen sensor (http://www.aadi.no/Aanderaa/Products/Sensors/OxygenOptode/default.aspx )

  - MBARI ISUS Nitrate sensor (Ken Johnson)

  - Wetlabs FL/BB Fluorometer/Backscatter (http://www.wetlabs.com/products/pub/specsheets/flbbap2ssa.pdf )

  - PAR Sensor (TBD)

  - GPS ( www.u-blox.com/en/gps-modules.html ). The GPS will not normally be sampled at depths other than zero or near zero.

  - 3 axis Accelerometer ( maybe http://invensense.com/index.html )