|  | Simulation of SAEfitting Date: Thursday, December 29, 2011 Designer: Hans Jannasch Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 4 Mesh Information 5 Study Results 7 Conclusion 10. |
| --- | --- |
| # Description SAEfitting-PNAtop |  |

# Assumptions

# Model Information Model name: SAEfitting Current Configuration: PNAtop. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet2; Solid Body; Mass:0.0496896 lb Volume:0.993791 in^3 Density:0.05 lb/in^3 Weight:0.0496559 lbf; \\vmware-host\Shared Folders\Dropbox\MBARI\APEX\APEX+pH\SAEfitting.SLDPRT Dec 29 12:37:20 2011.

# Material Properties Model Reference; Properties; Components. Name:; PEEK. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 1.03421e+008 N/m^2. Tensile strength:; 1.03421e+008 N/m^2.; SolidBody 1(Fillet2)(SAEfitting).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Pressure-1; Entities:; 12 face(s). Type:; Normal to selected face. Value:; 3000. Units:; psi..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.0998296 in. Tolerance; 0.00499148 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 13329. Total Elements; 7857. Maximum Aspect Ratio; 52.168. % of elements with Aspect Ratio < 3; 96.4. % of elements with Aspect Ratio > 10; 0.0636. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:03. Computer name:; TREMATOMUS64-VM.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 8.91209 psi Node: 1230; 7846.48 psi Node: 10211. SAEfitting-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 in Node: 171; 0.00181449 in Node: 1313. SAEfitting-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. SAEfitting-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 1.91169 Node: 10211; 1683.11 Node: 1230. SAEfitting-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion OK to 3000psi with safety factor of 2