Dwarf Core Board Setup and Testing

Outline:

- Program Xilinx PLD

- Program MSP430 microprocessor

- Home Motor0

- Test overcurrent Motor0

- Test positioning Motor0

- Test gripper control Motor0

- Home Motor1

- Test overcurrent Motor1

- Test positioning Motor1

- Test gripper control Motor1

**Programming**

- Set switches to “collection” setting (on/on/on/off/off/off)

- Set power supply to 3.3V

- Connect XILINX DLC7 to computer’s parallel port and PS/2 port or USB-PS/2 adaptor.  //Light on XILINX should be yellow

- Connect programming cable (white/black/red w/label: XILINX PGM CABLE) to core board J7

  - Connect end of programming cable with power leads to core board J7

  - Connect opposite end of programming cable  XILINX

  - Connect banana plugs to 3.3V power and turn on power supply output.  //XILINX light should turn green

- Start iMPACT

  - Select “load most recent project” (core.ipf)  Make sure “xrc3384xl u21 467c.jed” appears (if ‘core.ipf’ does not appear, navigate to: ProjectLibrary\ESP\2G\Project.Drawings\Project.EE.Dwgs\Programmable Logic\Core\Core_Ver1\Core\implement\ver1\rev6 and select core.ipf)

  - Right-click icon and select Program

    - Check Erase Before Programming

    - Check Verify

    - Check XPLA and type v1r6_467c in text box

  - Click OK

  - Right-click and select “Get Device Checksum” //should see 467c below

  - Right-click and select “Get XPLA Device UES”  //v1r6_467c below

- Connect Olimex MSP430 JTAG box to J7 of core board.

  - Press and hold left (red) “Write” button until the “Read” button blinks.  //the “write button should start blinking rapidly, and then the “read” button will blink several times more slowly

  - When all blinking stops, remove connector; board is programmed.

**Power up system with core board**

Set power supply voltage to 13V +/- 1V

Connect power supply to Sleepy J2  //Board should draw nominal 0.2A – 0.3A

Attach DwarfBus ribbon cable from Sleepy J6/J8/J10 to Core J3

Attach power cable from J5 on Sleepy to J2 on core board

**Login**

telnet esptech

esp

genosensor

esp //make sure that there are no errors (30sec load time)

**setup**

require ‘testDwarf’ //True

testDwarf

**Testing**

(This testing done with Dwarf firmware v3.68, ESP2 v3.00 I2C Gateway 3.01)

- Motor_0 Home and Test

  - Setup:

Attach Dwarf Test proto board to dwarf core board J8 (for Motor _0).

  - Home Sensor test // Proves home sensor works

    - CS.home.to :home  //Motor accelerates in one direction.

    - Place blade of screwdriver between hall sensor.

    - Motor slows slightly and stops.

    - Motor changes direction and accelerates.

    - Remove blade.

    - Motor slows, changes direction slightly, and stops.

  - Motor Test

    - CS.jog 30000  //Motor should ramp up in speed, slow down, then stop.

    - CS.jog -30000  //Motor should ramp up in speed, slow down, then stop in the opposite direction.  (If homed first, will indicate position as home on screen.)

- Motor_0 Overcurrent test

  - Connect DMM in circuit with 5ohm load resistor and 4-circuit connector housing (blue wire with clips and connector).  Set DMM to measure 2A.

  - Plug connector housing into motor0 (J8) pins 2 and 8.

  - CS.force 26 and CS.force -26  //Current should go to +/- ~0.5A

  - CS.force 50 and CS.force -50  //Current should go to +/- ~1.0A

  - CS.force 95 and CS.force -95 //Current goes to 0 and there should be an over-current error (note: will not get over-current error unless sensor is homed first)

  - Disconnect 4-circuit housing from J8

- Motor_0 Positioning Tests

  - Setup

    - Re-attach Dwarf Test proto board to dwarf core board J8 (for Motor _0).

  - Threshold Test

    - CS.config.threshold= :negative

    - CS.configure  //Green light comes on

    - CS.fill  //Motor should begin moving.

    - Press and release “Threshold” button

      - Motor will slow down

      - Motor will switch directions and go to threshold position.

    - CS.config.threshold= :off

    - CS.configure //LED should turn off

  - Limit switch test

    - CS.fill //Motor should begin moving.

      - Press and release “Limit” 5-7 button

      - Motor should stop abruptly.

      - Error message should be displayed.

    - Press Reset button (SW1) on core board.

    - CS.configure

    - CS.jog -30000 //Motor should begin moving.

      - Press and release “Limit” 1-3 button

      - Motor should stop abruptly.

      - Error message should be displayed.

    - Press Reset button (SW1) on core board.

    - CS.configure

  - Gripper Control Test

    - Connect DMM to “Test” and “Ground” terminals on Dwarf Test board, and set to measure VDC.

    - Set Jumper switch to “On” position.

    - Set Gripper switch to “Red” position.

    - Hand0.open

    - Voltage on DMM should be ~ 3.3V.

    - Set Gripper switch to “Green” position.

    - Voltage on DMM should be 0.0V.

    - Set Jumper switch to “Off” position. //(Status message of “opened” should be displayed.)

    - Set Jumper switch back to “On” position.

    - Leave Gripper switch in “Green” position.

    - Hand0.close

    - Voltage on DMM should be ~ 3.3V.

    - Set Jumper switch to “Off” position. //(Status message of “closed” should be displayed.)

- Motor_1 Home and Test

  - Setup:

Attach Dwarf Test proto board to dwarf core board J9 (for Motor _1).

  - Home Sensor test // Proves home sensor works

    - CC.home.to :home  //Motor accelerates in one direction.

    - Place blade of screwdriver between hall sensor.

    - Motor slows slightly and stops.

    - Motor changes direction and accelerates.

    - Remove blade.

    - Motor slows, changes direction slightly, and stops.

  - Motor Test

    - CC.jog 30000  //Motor should ramp up in speed, slow down, then stop.

    - CC.jog -30000  //Motor should ramp up in speed, slow down, then stop in the opposite direction.  (If homed first, will indicate position as home on screen.)

- Motor_1 Overcurrent test

  - Connect DMM in circuit with 5ohm load resistor and 4-circuit connector housing (blue wire with clips and connector).  Set DMM to measure 2A.

  - Plug connector housing into motor1 (J9) pins 2 and 8.

  - CC.force 26 and CC.force -26  //Current should go to +/- ~0.5A

  - CC.force 50 and CC.force -50  //Current should go to +/- ~1.0A

  - CC.force 95 and CC.force -95 //Current goes to 0 and there should be an over-current error (note: will not get over-current error unless sensor is homed first)

  - Disconnect 4-circuit housing from J

- Motor_1 Positioning Tests

  - Setup

    - Re-attach Dwarf Test proto board to dwarf core board J9 (for Motor _1).

  - Threshold Test

    - CC.config.threshold= :negative

    - CC.configure  //Green light comes on

    - CC.unsealed  //Motor should begin moving.

    - Press and release “Threshold” button

      - Motor will slow down

      - Motor will switch directions and go to threshold position.

    - CC.config.threshold= :off

    - CC.configure //LED should turn off

  - Limit switch test

    - CC.closed //Motor should begin moving.

      - Press and release “Limit” 5-7 button

      - Motor should stop abruptly.

      - Error message should be displayed.

    - Press Reset button (SW1) on core board.

    - CC.configure

    - CC.jog -30000 //Motor should begin moving.

      - Press and release “Limit” 1-3 button

      - Motor should stop abruptly.

      - Error message should be displayed.

    - Press Reset button (SW1) on core board.

    - CC.configure

  - Gripper Control Test

    - Connect DMM to “Test” and “Ground” terminals on Dwarf Test board, and set to measure VDC.

    - Set Jumper switch to “On” position.

    - Set Gripper switch to “Red” position.

    - Hand1.open

    - Voltage on DMM should be ~ 3.3V.

    - Set Gripper switch to “Green” position.

    - Voltage on DMM should be 0.0V.

    - Set Jumper switch to “Off” position. //(Status message of “opened” should be displayed.)

    - Set Jumper switch back to “On” position.

    - Leave Gripper switch in “Green” position.

    - Hand1.close

    - Voltage on DMM should be ~ 3.3V.

    - Set Jumper switch to “Off” position. //(Status message of “closed” should be displayed.)