Deep ESP 2011 Juan deFuca Deployment July 2011

TO DO LIST

Deep ESP Elevator Equipment

Sphere:  	Gordon

MFB

On elevator:	DWSM

CTD

ISUS

ISMS

WAND

AC Modem

SAT Beacon, RF Beacon, Strobe, Pinger

ISMS:		Send back to Harvard?

Get dimensions and mass of other housing, recalc weight balance

Make new mounts

Modify cabling?

Receive new pressure housing

Assemble, and Test

Mount on elevator

ISUS:		Receive fittings and probes

MinK’s in

P Caps in

Relief Valves ordered

P Test covers in shop

Orings/seals in

Order new probes

Assemble 4000m housings

Rosal group mounting MinK’s

Leak and hydro test

Mod ESP/ISUS cables

Wire ISUS inside new can

Calibrate ISUS for sulfides

Assemble ISUS for deployment

Leak test, check ground isolation.

Mount on elevator

WAND:	Have functional reqmts? Comments?

Tested optional pumping system, review options.

Finalize schematic

Order components

Design parts

Assemble system

Convert 1 to Right Handed

Test in tank

Measure flowrate, adjust. ~2 L/min, each path

Inspect temp can vent port, find right fitting to seal. Replaced end cap

HPI:		Order parts

Replace pump, valve, hoses, etc

Proof test HPI

Set relief valves

DWSM: 	New jacket bolt torque tool/method

Evaluate open rings for new pressure…

Compliance bags issues

Proof test DWSM for JdeF depth operation

Order Supernuts

Inspect IEP Pump

Fix Sample bag switches

Rebuild sample switches

Assemble and test Sample side

Inspect Resample bag and switches

IEP accumulator?

Designed

Parts to shop

Assemble

Test

Install

System test, Low pressure

High Pressure test.

Adjust IEP volume

Validate IEP volume

Test IEP more

Fix HPP encoder

Check with mfg what damage may be from

Review HPP drive methodology

SPHERE:	New isolation valves

New valves ordered

Test and validate new valve tree (including overpressure valves)

Add heat exchangers to heater ring

Heat exchangers ordered

Dogleg to machine shop

Replace purge plugs

ELEVATOR: Rewire/cast Battery Combiner box

Wet mate release for recovery, ROV/tray pack

Satellite beacon

Ordered

Mounted

Tested

Decision on burnwire vs ROV only release.  Burnwire

SHIP:		Check on WF Hull transducer

Get Rover modem and cable

Run cable thru ship

Review weight ring for dunking transducer

Review tow point/recover lines with WF crew

Test comms to ROV on June 20, when ROV is on dock. 10m and 70m tethers

CHECKLIST:

In trailer work

Proof test system to JdeF depth (2000m)

Remove Gordon

Modify sphere components

Modify DWSM?

Replace water bags?

Test bag switch points

Flush/clean HPI

Core process, develop array, PCR?

Full cycle test Under HP

CTD/ISUS in a bucket

Chill Down to 4C

Full cycle cold test.

Test ISMS in lab

Can ISMS

Canning Gordon

Add hobo’s

Load reagents

Load Pucks

Empty wastes

check ground isolation.

Sphere isolated 500kohm

Install sphere insulation

Close Sphere

Torque bolts, cross pattern  (75 in-lbs)

Go-no go gauge test

Leak test and N2 purge.

Clean and reset sphere relief (5 turns out, 4 psi)

Can DWSM

Leak test DWSM bells

Clean, set Bell relief ports: (3 turns out)

Return to trailer

Chill Down to 4C

Post CAN runs

Elevator and Foam pack to High Bay

Install Drop Weight Release

Mount Dummy Jack

Rebuild battery boxes (8)

Mount Battery Combiner box

Mount 8 Batteries on elevator

Install ISMS

Install Wand temp can

Wand holder brackets

Add foam to pack

Add ROV push bumpers to Foam pack

Install Ac. Modem

Install Iridium beacon

Install Pinger, Strobe, etc.

Attach bumper rings

Attach wand, wind up

Test Tank

Weigh, trim and balance

Test burn wire release

Test voltage to burnwire after battery change.

Test battery tray separation (performed dry, 40-50 lbs)

Test Fluidic Stream (Sample point/DWSM/ISMS/ISUS/CTD…)

Test manual release

Modem Comm

Back to highbay

Add feet to wand bases

Remove foam pack

Change Ac modem batteries

Change CTD batts

Re-Install CTD

Back to trailer, chill down

Post Can QC

Back to highbay

Add feet to wand bases

Remove foam pack

Install 4m tether, attach yale grips

Attach tether anchor at elevator

Attach yale grip to wetmate handle

Install Super sub mini  (on WF)

Change Ac modem batteries

Change CTD batts

Re-Install CTD

Twist CTD lid, re-attach

Install ISUS

Remove foam. (1 brick)

Test voltage to burnwire after battery change.

Prep for deploy

Make lists of equipment, tools, spare, etc.

Pack it all

Clean/Mop trailer

Load Trailer

Anchor equip in trailer

Verify generator and trailer compatibility

Set trailer temp to 11C for shipping

Prior to deploy

Attach super sub mini beacon

-------------------------------------------------------------------

Wand Pump only 2L/65 sec

Wand Pump + CTD Pump 2L/52 sec

Elev Pump only 2L/60 sec

Elev Pump + CTD Pump 2L/50

Super submini: 4.5 kg air 2.4 kg water, 10 lbs air 5 in water

70m tether w/ wetmate:  46 lb in air, 16 in SW

3m tether w/ wetmate: 7lbs in air, 3 in SW

ISUS Ti housing (empty) 16 lb in air.

Test weight 4334 in air, 100 in SW