Beam Calculations for Top and Bottom Cross Members

Applied Force    = 1000 lbs

Length           = 32 in

Tube width       = 2 in

Tube height      = 4 in

Tube thickness   = 0.25 in

Material Modulus = 30000000 psi

Cross sectional area   = 2.75 in^2

2nd Moment of area     = 5.3073 in^4

Maximum Bending Moment = 8000 in-lb

Maximum Shear Force    = 500 lbs

Maximum Deflection     = -0.0042876 in

Maximum Stress due to Bending = 3014.7203 psi

Maximum Stress due to Shear   = 181.8182 psi

Beam Calculations for Vertical Legs

Applied Force    = 500 lbs

Length           = 77 in

Tube width       = 3 in

Tube height      = 2 in

Tube thickness   = 0.1875 in

Material Modulus = 30000000 psi

Cross sectional area   = 1.7344 in^2

2nd Moment of area     = 1.0613 in^4

Maximum Bending Moment = 9625 in-lb

Maximum Shear Force    = 250 lbs

Maximum Deflection     = -0.14936 in

Maximum Stress due to Bending = 9068.7216 psi

Maximum Stress due to Shear   = 144.1441 psi

Beam Calculations for Lift Bar

Applied Force    = 1000 lbs

Length           = 28 in

Tube width       = 2 in

Tube height      = 3 in

Tube thickness   = 0.1875 in

Material Modulus = 30000000 psi

Cross sectional area   = 1.7344 in^2

2nd Moment of area     = 2.0506 in^4

Maximum Bending Moment = 7000 in-lb

Maximum Shear Force    = 500 lbs

Maximum Deflection     = -0.0074341 in

Maximum Stress due to Bending = 5120.4572 psi

Maximum Stress due to Shear   = 288.2883 psi