July 2011, Juan De Fuca deployment

Deep ESP

The Juan De Fuca Power Company

Use scenario:

Deployment Day

Sled #1 and #2 batteries fully charged

Elevator lowered

ROV dives with Sled #1

Elevator located and moved into position

ROV plugs in and Science begins monitoring instrument

Upon departure procedure:

- Unplugs

- Unloads Sled

- Plugs sled to elevator

- Enables battery

- ROV departs

Recover ROV

Sled #2 loaded onto ROV

ROV available for science operations

Days during deployment

ROV dives with battery sled and locates elevator

ROV disables deployed battery pack

Sleds are swapped

ROV plugs into elevator and science monitors

Upon departure procedure

Recover ROV

Charge batteries while on ROV

ROV available for science operations

Functional Requirements:

2 battery sleds

48V nominal

100 Amp-hour

Suggest using this cell: http://www.mrsolar.com/pdf/MK_Battery/MK8G31.pdf

Suggest individually packaged using Okuda’s method

MARS side style ODI connector (Socket), ESP is supplying one per sled and one per ROV, 3 total.

ODI Make and Break must be done with power off (Battery On/Off will be while fully loaded)

Support 15 amp continuous draw

Batteries are individually chargeable on deck (4 chargers)

Sled removable and left on bottom

ODI with ability for elevator to pull apart (added)

ESP will supply chargers

Each battery sled has a pinger

ESP will purchase supplies including batteries