Pull mooring only onto deck

Mooring lift line

Lifting Line

Float

Screw Pin Shackle

3 tie wraps

Dive Bag

2 Bale and hairpin cotter pins

Boat pool with mooring hook

Happy Hooker (Aaron did this get replaced?)

Tag Lines

Tie downs

General mooring inspection (cable, cotters, shackles, lines ...)

Replace instrument

¾” socket

½” ratchet

¾” open und wrench

½” socket

½” open end wrench

Tie wraps

Replace instrument batteries

½” socket

½” ratchet

2 Alkaline DSPL

Tie wraps

Replace Iridium modem battery

½” deep socket

½” extension

½” ratchet

Iridium battery pack

Replace ISUS battery

Regular large screw driver

5/16” socket

ISUS battery pack

Gloves

Replace ISUS screen

ISUS screen assembly

Clean ISUS sensor tip

Tooth Brush

Alcohol

Inspect bungees (bring spare along)

Bungee Assembly

CTD dismount and Replace batteries

½” Socket

½” ratchet

½” open end wrench

Large adjustable wrench

9 D-cell alkaline batteries

Unsure what else to get batteries out?

O-Ring set for CTD

Parker O-Ring Lube

Remount CTD Bundle up and strap all cables

½” Socket

½” ratchet

½” open end wrench

Tie wraps

Test communications to instrument

Away communications bag

Radio link to shore

Test communication with CTD

Away communications bag

Radio link to shore

Test communications with ISUS

Away communications bag

Radio link to shore

Recover CTD data and then Clear memory

Away communications bag

Radio link to shore

Recover ISUS data and then clear memory

Away communications bag

Radio link to shore

Confirm CTD and ISUS sample rate is 20 minutes

Away communications bag

Radio link to shore

Confirm GPS report received

Someone on shore

Take deep breath and think if I missed anything

Brain

Deploy refurbished instrument

Shoot

Quick Release

Cotter Pins

Pliers

1-1/8” open end wrench set

Float

Dive Bag

Confirm depth and begin mission Go home

Other things

Cotter pins

Screw pin shackle

Howto CTD and ISUS

Time: 6:30 for 7:00

Divers: Mike and Mike