ESP Sampler Firmware

Version 0.0 – initial release

0.1 – added Decon pump to list of motors

Motors

Cylinder

Arm

Clamp

Reagent pump

Decon pump

Solenoids

Intake

N2

RNA

Waste

Decon

Vacuum Select

Vacuum Break

Exhaust

Vacuum pump

Sensors

Intake pressure (Wheatstone)

Vacuum switch

N2 pressure

RNA Scale (Wheatstone)

Decon Scale (Wheatstone)

Flow

Opto-interrupter

Force

Cylinder present

Interfaces

Command

NFC

Sample pump

Motors controlled by Allegro A4988

https://www.allegromicro.com/-/media/files/datasheets/a4988-datasheet.pdf

Step and Direction with sleep

Timer and capture compare

Opto-interrupter or Force for location

Solenoids actuated by Analog MAX22200

https://www.analog.com/media/en/technical-documentation/data-sheets/MAX22200.pdf

SPI

NFC currently a NXP PN532 but not recommended for new designs

https://www.nxp.com/docs/en/nxp/data-sheets/PN532_C1.pdf

Looking for similar with a serial or SPI interface

Sample pump currently TBD

Possibly a contact closure or a power and serial interface

Wheatstone

HX711 or TI ADS1234

https://cdn.sparkfun.com/assets/b/f/5/a/e/hx711F_EN.pdf

https://www.ti.com/lit/gpn/ADS1234

SPI or Bit-Bang serial

Flow

Paddle wheel flow and rate sensor.

https://assets.omega.com/pdf/test-and-measurement-equipment/flow/turbine-flow-meters/FTB300_Series.pdf

Pulse output.

Pulse rate = flow rate

Pulse count = volume

Motor Trajectory

Simple constant acceleration (see figure)

Plateau Pace

Final Pace begins a specified amount before and after terminating event

Tail Pace after event for a specified count ends motion

Pace is inverse of velocity and is counts of timer between steps

Event may be a state from

opto-interrupters for Arm and Cylinder or,

Force sensor for Clamp

Low level hard coded basic functions such as:

Motor moves

Simple number of steps and fixed Pace, sleep

Trajectory estimated steps, Landing zone, event state, after event steps, sleep

Trajectory estimated steps, Landing zone, event force, under or over, after event steps, sleep

Solenoid

On and errors

Off and Errors

Multiple on at same time

One per command

All off command

NFC

ID puck

Read puck

Type

Serial number

Write Puck

Sample volume

Terminating rate

Start time

Stop Time

Sample temperature

Flow rate and volume

current

User Interface

Command Line

Asynchronous

Serial

Non-Volatile scripts/functions/routines

Micro-Python a possibility

Changeable without compiling or flashing

Example:

Sample tube,filter type,volume,min rate,max sample time,purge time

Decon volume,time

Status Responses:

Sample begin

Loading puck

Cylinder to tube 1

Arm to cylinder

Vacuum reached

Arm to clamp

Clamp to pinch

Puck release

Clamp to clamped

All motion axis should use incremental moves.  Optical sensors have transitions between states at the various required positions.  Referring to the Axis Trajectory Profile figure, the estimated steps is a predefined number from the current position state to the requested state.  The end of the trajectory has a constant Pace that is specified and starts Landing Zone steps before the estimated move steps.  This constant pace may continue beyond the estimated steps by Landing Zone steps.  This constant Pace phase is terminated be either the requested state being observed or Landing Zone steps beyond Estimated Steps has been reached.

The trajectory accelerates the pace to a minimum value.  This value

Cylinder

Is a round object that is like the cylinder of a revolver.  The cylinder holds the pucks for sampling.  It consists of 12 tubes which each hold 12 pucks.  The cylinder is located with a set of 6 opto-interrupters that provide a transition at the center of each tube.  Therefore, there are 12 valid states of the 64 possible from the 6 sensors.  The cylinder may rotate in either direction infinitely rotating.  should be positioned for a tube in one direction.

Arm

The main puck manipulation axis with one degree of freedom, rotation.  It has a vacuum enabled end effector with a vacuum switch to detect when a puck has been acquired.  The rotation is located with a set of 4 opto-interrupters that provide a transition at the center of or near each position.  When a near position transition is reached a fixed number of steps beyond is required to reach the ultimate destination.

Clamp

Is a C-clamp shaped structure that makes the fluidic connection to the puck.  It has a single degree of freedom that is located using a force sensor.  The clamp is moved estimated steps until a force level is reached for either a above or a below force.