October 28, 2013

Protocol: FilterEvacuate w/ air) Add ReagentEvacuate w/air

Parts List:

Accel Puck-Fixture Combination (Fall 2013 version)

Accel Puck Version 4, cut gasket w/ amber plastic or Ti top

0.2um filter, 35psi pressure on intake, through torrid valve

V4 puck Fixture

(Figure 1:  2-way stopcocks on intake, accumulator, and exhaust ports)

(Note: Below protocol cannot be run in current archive cartridge version)

Actuation Changes

- Utilizing the diaphragm as an exit port and valve (=Accumulator)

- Independent opening/closing of the intake and exhaust

- Independent opening/closing of accumulator valve

Figure 1

- Assemble fixture and place fixture on angle to position the accumulator outlet at the location where the air bubble accumulates as the puck is being filled.

- Filtration (All valves closed)

  - Intake, exhaust and accumulator valves closed.  Pressure pot at 30psi off.

  - Hook up lines to intake and exhaust.

  - Open pressure pot valve. Check for leaks.

  - Open exhaust valve

  - Open intake valve

  - Measure the filtration rate to at least 300ml volume

  - Turn off intake valve

  - Turn off pressure pot valve

- Evacuation  (Intake and accumulator valves closed, exhaust open)

  - Remove intake line

  - Attach a 3cc syringe containing 1.5ml air to the intake

  - Hold syringe firmly.  Open intake valve to deliver 1.5ml air to puck, with syringe bottomed out close the valve

  - Repeat b-c for a total of 4, 1.5 ml pushes

- Reagent delivery (Intake and accumulator valve closed, exhaust open)

  - Slowly open the accumulator valve to vent off the air pressure

  - Attach 3cc syringe containing 1.5ml of reagent to Intake valve

  - Open intake valve (There should be no backpressure on the syringe.  All valves on fixture are open)

  - Slowly deliver 1.4 ml of the reagent to puck.

  - Close the accumulator and exhaust valves.

  - Deliver the remaining fluid in the syringe.

  - Close intake when the syringe bottoms out.

  - Incubate X min at X temperature (Dependent on lysis or fix procedure)

- Evacuation and Reagent recover

  - Repeat evacuation steps b-d, but with the exhaust tube positioned in a 2ml tube to recover the reagent