Cartage Assembly Procedure: 9/9/13:

Tools Required:

- 2mm Allen wrench

- 1.5mm Allen wrench

- 0.9mm Allen wrench

- O-ring tool

- Snap ring pliers or tweezers

Assembly Instructions:

- Put 6 1X3mm O-rings on plunge valve piston in the grooves

- Put 2 1X6mm O-rings each (4) short titanium plungers

- Press plungers into syringe block

- Insert snap rings into syringe block

- Place one 1X17mm O-ring  and 1X9.5mm O-ring on the cover

- Slide cover onto the puck cassette block

- Insert one  1/16th dowel pin into puck cassette block

- Place 1X13mm and 1X3mm O-ring on evac. manifold wedge

- Place wedge on puck cassette block

- Place two 1X11mm O-rings on syringe manifold wedge

- Place syringe manifold wedge on evac. Manifold wedge

- Place four 1X6mm o rings on

- Insert second dowel pin

- Press syringe block and puck cassette block together

- Screw together with M2.5 screws

- Insert plunge valve piston from the bottom up (so the O-rings go in last)

- Screw in M2 set screw

- Set puck assembly into puck cassette block

- Slide puck bracket into place

- Screw puck bracket in from the side first

- Then screw in from the bottom